HypPy for QGIS

Complete Workflow Guide

Version 2.45  |  March 2026

# 1. Overview

The HypPy for QGIS plugin provides hyperspectral analysis tools accessed via the QGIS Processing Toolbox and the HypPy toolbar dropdown. The toolbox is organised as follows:

- 1 - Workflow Guide  —  Open this guide in your browser

- 2 - Data Import  —  Build overviews for fast display; spatial/spectral subset

- 3 - Generate WoM Rasters  —  Generate WoM rasters and wavelength maps

- 4 - Decision Tree Classification  —  Classify minerals and physical properties

- 5 - SAM Classification  —  Spectral Angle Mapper classification

- 6 - Spectral Processing Tools  —  Log Residuals, PCA, filters, band maths, continuum removal

- 7 - Feature Extraction  —  One-click WoM + wavelength map + legend per absorption feature

- 8 - Convert Tools  —  Format conversion utilities

## Plugin Structure

| Algorithm | Toolbox Group | Purpose |
| --- | --- | --- |
| Step 1 - Open Workflow Guide | 1 - Workflow Guide | Open this guide in your browser |
| D01 - Build Overviews (Fast Display) | 2 - Data Import | Build GDAL overview pyramids for fast display |
| D02 - Subset | 2 - Data Import | Extract a spatial and/or spectral subset |
| Step 2a - Generate All Three WoM Rasters | 3 - Generate WoM Rasters | Generate WoM-A, WoM-B and WoM-C in one step |
| Step 2a - Generate WoM-A (750–1300 nm) | 3 - Generate WoM Rasters | Generate WoM-A individually |
| Step 2a - Generate WoM-B (1850–2100 nm) | 3 - Generate WoM Rasters | Generate WoM-B individually |
| Step 2a - Generate WoM-C (2100–2400 nm) | 3 - Generate WoM Rasters | Generate WoM-C individually |
| Step 2b - Wavelength of Minimum (custom) | 3 - Generate WoM Rasters | Extract absorption features for any range |
| Step 2c - Wavelength Mapping | 3 - Generate WoM Rasters | Create RGB visualisation from WoM output |
| DT Mineral 2100–2400 | 4 - Decision Tree Classification | Classify minerals in 2100–2400 nm range |
| DT Albedo | 4 - Decision Tree Classification | Classify brightness per pixel |
| DT Fedrop | 4 - Decision Tree Classification | Classify iron drop intensity |
| DT Illcryst | 4 - Decision Tree Classification | Classify illite crystallinity |
| DT Ill-Kaol | 4 - Decision Tree Classification | Classify illite vs kaolinite ratio |
| DT Kaol-Cryst Classification | 4 - Decision Tree Classification | Classify kaolinite crystallinity using 2162/2206 nm depth ratio |
| DT Dick-Kaol Classification | 4 - Decision Tree Classification | Classify dickite vs kaolinite using 2162/2178 nm shoulder wavelength |
| DT Kandite Group Classification | 4 - Decision Tree Classification | Classify all five kandite group minerals (kaolinite, dickite, nacrite, halloysite, metahalloysite) |
| DT Pyrophyllite Classification | 4 - Decision Tree Classification | Classify pyrophyllite using 2066 nm doublet and 1396 nm OH feature |
| DT Mineral Map | 4 - Decision Tree Classification | 17-class mineral map from WoM-B + WoM-C |
| Spectral Angle Mapper (SAM) | 5 - SAM Classification | Classify pixels against a spectral library |
| Convex Hull Removal | 6 - Spectral Processing Tools | Continuum removal using convex hull fitting |
| Band Ratio | 6 - Spectral Processing Tools | Single-band ratio by wavelength |
| Band Ratios (Sequential) | 6 - Spectral Processing Tools | All-band sequential ratios with delta |
| Band Depths | 6 - Spectral Processing Tools | Per-band absorption feature depths |
| Band Math | 6 - Spectral Processing Tools | NumPy expression over image bands |
| Spectra Math | 6 - Spectral Processing Tools | Per-pixel spectral expression |
| Log Residuals | 6 - Spectral Processing Tools | Log Residuals / Kwik Residuals normalisation |
| PCA — Forward | 6 - Spectral Processing Tools | SVD-based PCA; outputs PC score image + .pca_stats file |
| Linear Filter (3×3 Convolution) | 6 - Spectral Processing Tools | 3×3 spatial convolution filter with presets and custom kernel |
| 3D Median / Mean Filter | 6 - Spectral Processing Tools | 3D spatial-spectral median or mean denoising filter |
| E09 - Zonal Statistics | 6 - Spectral Processing Tools | Per-zone per-band statistics written to .txt files |
| FE01–FE18, FE-REE | 7 - Feature Extraction | One-click WoM + map + legend per feature |
| Convert Image to ENVI | 8 - Convert Tools | Convert TIFF/JPEG/PNG to ENVI flat-binary |
| Convert EOS HDF to ENVI | 8 - Convert Tools | Extract EOS HDF subdatasets to ENVI |
| Convert ASTER HDF to ENVI | 8 - Convert Tools | Extract ASTER HDF bands to ENVI |
| Convert EMIT L2A to ENVI | 8 - Convert Tools | Convert EMIT L2A NetCDF4 to ENVI |

# 2. Installation

Copy the HypPy_for_QGIS folder to your QGIS plugins directory:

- Windows:  %APPDATA%\QGIS\QGIS3\profiles\<profile>\python\plugins\

- Linux:  ~/.local/share/QGIS/QGIS3/profiles/<profile>/python/plugins/

- macOS:  ~/Library/Application Support/QGIS/QGIS3/profiles/<profile>/python/plugins/

Restart QGIS, then enable the plugin via Plugins > Manage and Install Plugins. The HypPy tools appear in the Processing Toolbox under the HypPy provider.

## Requirements

- QGIS 3.16 or later  (QGIS 4.x also supported)

- numpy  — bundled with QGIS

- GDAL/OGR  — bundled with QGIS

- scipy  — required for Spectra Math and Band Math. Bundled with most QGIS installations. If missing: Windows (OSGeo4W Shell): pip install scipy  |  Linux / macOS: pip3 install scipy

# 3. Data Import

The Data Import group contains tools for preparing a hyperspectral image before analysis. The recommended starting workflow for any new dataset is: Build Overviews first so QGIS can navigate the image quickly, then Subset if you only need to process a portion of the scene or a restricted wavelength range.

## 3.1  D01 — Build Overviews (Fast Display)

Builds GDAL overview pyramids so that QGIS can display large hyperspectral images quickly at any zoom level without loading the full file into memory. On completion the image is automatically loaded into the QGIS map canvas with -9999 pixel values set to transparent.

Why this is needed:  Large hyperspectral files (such as AVIRIS at ~7 GB stored in BIL interleave) can take a very long time to load in QGIS. Without overviews, QGIS reads full-resolution data even when zoomed far out. Building overviews is a one-time operation that makes subsequent loading and navigation fast.

| Parameter | Description |
| --- | --- |
| Input Image | The hyperspectral image — point to the .hdr or the binary data file |
| Overview Levels | Standard (2–32) suits most images. Extended (2–128) for very large files. Coarse (4–64) builds faster. |
| Resampling Method | average (default) gives best quality for reflectance data. nearest is fastest for classified outputs. |
| External overviews | Recommended (default on). Writes a .ovr sidecar; original file is not modified. |

Tip: Building overviews on a 7 GB AVIRIS file takes a few minutes — a one-time cost. The .ovr file must remain beside the image. For ENVI BIL files, converting to BSQ with D02 Subset before building overviews gives the best display performance for RGB composites.

## 3.2  D02 — Subset

Extracts a spatial and/or spectral subset from an ENVI hyperspectral image, writing a new ENVI flat-binary output. Use this tool to clip the scene to your area of interest or to restrict the spectral range before processing — both operations reduce file size and speed up subsequent algorithms significantly.

### Spatial subset modes

Pixel mode (default — "Use map coordinates" unticked): define the region as pixel row/column offsets from the top-left corner of the image.

Map coordinate mode ("Use map coordinates" ticked): define the region using the same coordinate system as the image (e.g. UTM eastings and northings). The tool converts map coordinates to the nearest pixel boundary automatically.

Bug fix (v2.44): Using "Use map coordinates" with "Use layer extent" previously caused a crash on QGIS 4.x due to a race condition between the layer extent background task and the GDAL dataset reader. This is resolved in v2.44 — the extent is now resolved before the dataset is opened.

| Parameter | Description |
| --- | --- |
| Input Image | ENVI hyperspectral image (.hdr or binary) |
| Use map coordinates | Tick to enter spatial bounds in map units rather than pixels |
| Spatial bounds | Start/end row and column (pixel mode) or X/Y min/max (map mode) |
| Start / End Wavelength (nm) | Spectral range to retain (0 / 99999 = all bands) |
| Output ENVI image | Path for output flat-binary; matching .hdr written automatically |

# 4. Generate WoM Rasters

This group generates the three Wavelength of Minimum (WoM) rasters required by the decision tree classifiers, and provides tools for custom wavelength range analysis and wavelength mapping. Run the WoM generation step first whenever processing a new hyperspectral dataset.

Use Step 2a - Generate All Three WoM Rasters to produce WoM-A, WoM-B, and WoM-C in a single operation, or run the individual algorithms if only specific ranges need to be regenerated.

## WoM Raster Specifications

| Name | Wavelength Range | Used by | Notes |
| --- | --- | --- | --- |
| WoM-A | 750 – 1300 nm | Future classifiers | Iron oxide and hydroxyl features |
| WoM-B | 1850 – 2100 nm | dt_illcryst, dt_ill_kaol, dt_mineral_map | Water and carbonate shoulder; illite crystallinity denominator |
| WoM-C | 2100 – 2400 nm | dt_mineral_map, dt_2100_2400, dt_fedrop, dt_illcryst, dt_ill_kaol | Primary clay mineral absorption region |

All Step 2a algorithms use Division continuum removal, extract 3 features per pixel by default (1–9 selectable), and honour the Bad Band List if present. The output raster has 2 × N bands: wavelength and depth for each feature, sorted deepest first.

## 5. Wavelength of Minimum (Custom Range)

Analyses each pixel's spectrum within a user-defined wavelength range. Applies convex hull continuum removal, locates local minima, and fits parabolas for sub-band-resolution wavelength and depth values. Use this when you need a WoM raster for a range other than the three presets in Step 2a.

| Parameter | Typical Value | Notes |
| --- | --- | --- |
| Input Image | Hyperspectral raster | Must have wavelength metadata |
| Start Wavelength (nm) | 2000 | Begin analysis window |
| End Wavelength (nm) | 2500 | End analysis window |
| Continuum Removal | Division | Division recommended for minerals |
| Number of features | 1–3 | 1 for single mineral, 3+ for multi-mineral |
| Broad feature fitting | Off | On for broad absorption features |
| Use Bad Band List | On if available | Excludes noisy bands flagged in ENVI metadata |

Output: A multi-band GeoTIFF containing wavelength/depth pairs for each requested feature. Band 1 = interpolated min. wavelength, Band 2 = interpolated depth. Features are sorted by depth (deepest first).

## 6. Wavelength Mapping

Takes a Wavelength of Minimum output and creates an RGB image. Colour (hue) represents absorption wavelength position and brightness represents absorption depth.

| Parameter | Typical Value | Notes |
| --- | --- | --- |
| Input (Step 2a or 2b output) | WoM GeoTIFF | Uses Band 1 and Band 2 directly |
| Min Wavelength (nm) | 2100 | Blue end of colour stretch |
| Max Wavelength (nm) | 2350 | Red end of colour stretch |
| Min Depth | 0.0 | Lower bound of brightness stretch |
| Max Depth | 0.2 | Adjust for brightness (try 0.05–0.3) |

Wavelength Map RGB: 3-band GeoTIFF. Hue encodes wavelength position (blue→green→yellow→red) and brightness encodes absorption depth.

Legend PNG: A 2D colour legend with wavelength (nm) on the Y axis and depth on the X axis. Displayed automatically in a floating dock window after processing. The dock is resizable by dragging its edges.

# 7. Feature Extraction

The Feature Extraction tools are one-click pipelines combining Wavelength of Minimum, Wavelength Mapping, and legend display into a single operation. Each tool targets a specific known absorption feature with pre-configured wavelength ranges and map stretch values. Running any Feature Extraction tool produces three outputs: a WoM raster, an RGB wavelength map, and a colour legend PNG — and automatically displays the legend in a floating dock window.

Each tool is named after its characteristic absorption wavelength and feature type: W suffix = water or hydroxyl combination band; D suffix = diagnostic mineral absorption feature.

| Tool | WoM Range | Map Stretch | Characteristic Feature | Reference |
| --- | --- | --- | --- | --- |
| FE01 - 1384D | 1350–1450 nm | 1375–1425 nm | Dickite OH doublet ~1384/1418 nm | Laukamp et al. (2021) Table 5 (1400D/W) |
| FE02 - 1396W | 1350–1450 nm | 1388–1406 nm | Pyrophyllite OH ~1396 nm | Laukamp et al. (2021) Table 5 (1400D/W) |
| FE03 - 1400D | 1350–1450 nm | 1387–1445 nm | Kaolinite / all OH-bearing sheet silicates ~1400 nm | Laukamp et al. (2021) Table 1 (1400D) |
| FE04 - 1480W | 1440–1520 nm | 1471–1491 nm | Water / hydroxyl combination band ~1480 nm | Laukamp et al. (2021) Table 5 (1480D/W) |
| FE05 - 1550W | 1510–1610 nm | 1520–1563 nm | Hydroxyl overtone / organic ~1550 nm | Laukamp et al. (2021) Table 5 (1550D/W) |
| FE06 - 1760W | 1730–1790 nm | 1751–1764 nm | Carbonate / organic combination ~1760 nm | Laukamp et al. (2021) Table 5 (1760D/W) |
| FE07 - 1900W | 1850–1960 nm | 1870–1940 nm | Halloysite H2O ~1900 nm | GMEX halloysite reference |
| FE08 - 2066D | 2050–2110 nm | 2055–2095 nm | Pyrophyllite doublet ~2066 nm | Laukamp et al. (2021) Table 5 (2160D) |
| FE09 - 2080D | 2060–2100 nm | 2075–2090 nm | Carbonate / AlOH diagnostic ~2080 nm | Laukamp et al. (2021) Table 5 (2080D) |
| FE10 - 2160D | 2138–2179 nm | 2159–2166 nm | Kaolinite / dickite diagnostic ~2160 nm | Laukamp et al. (2021) Table 5 (2160D) |
| FE11 - 2178D | 2138–2210 nm | 2162–2196 nm | Dickite AlOH shoulder ~2178 nm | Laukamp et al. (2021) Table 5 (2200D/W) |
| FE12 - 2200W | 2120–2245 nm | 2185–2215 nm | AlOH — illite / smectite / kaolinite region | Laukamp et al. (2021) Table 5 (2200D/W) |
| FE13 - 2206W | 2120–2245 nm | 2197–2215 nm | Kaolinite AlOH ~2206 nm | Laukamp et al. (2021) Table 5 (2200D/W) |
| FE14 - 2250W | 2230–2280 nm | 2248–2268 nm | AlOH / MgOH transition ~2250 nm | Laukamp et al. (2021) Table 5 (2250D/W) |
| FE15 - 2290W | 2270–2320 nm | 2279–2338 nm | MgOH / carbonate ~2290 nm | Laukamp et al. (2021) Table 5 (2290D/W) |
| FE16 - 2320W | 2295–2345 nm | 2300–2340 nm | Carbonate 3ν₃CO₃ / Mg-chlorite ~2320 nm | Laukamp et al. (2021) Table 5 (2320D/W) |
| FE17 - 2350W | 2310–2370 nm | 2320–2366 nm | Carbonate / chlorite ~2350 nm | Laukamp et al. (2021) Table 5 (2350D/W) |
| FE18 - 2390W | 2375–2435 nm | 2377–2406 nm | MgOH / chlorite / serpentine ~2390 nm | Laukamp et al. (2021) Table 5 (2390D/W) |
| FE-REE - Neodymium VNIR | 530–910 nm | 560–900 nm | Nd³⁺ absorptions ~580/740/800/870 nm (all four bands) | Turner et al. (2014–2018); Boesche et al. (2015) |

Workflow Tip: Feature Extraction tools are designed for rapid reconnaissance. Run several tools to identify which absorption features are present and spatially coherent before committing to a full decision tree classification workflow.

# 8. Decision Tree Classification

The decision tree classifiers use pre-computed wavelength analysis outputs to assign mineral or physical property classes to each pixel.

| WoM | Wavelength Range | Used by |
| --- | --- | --- |
| WoM-A | 750–1300 nm | Future classifiers |
| WoM-B | 1850–2100 nm | dt_mineral_map, dt_illcryst, dt_ill_kaol (depth denominator) |
| WoM-C | 2100–2400 nm | dt_mineral_map, dt_2100_2400, dt_fedrop, dt_illcryst, dt_ill_kaol |

### DT Mineral 2100–2400

Classifies 16 mineral classes including kaolinite, muscovite, illite, pyrophyllite, alunite, and carbonates.  Inputs: WoM-C only.

### DT Albedo

Classifies pixels into four brightness classes dynamically using quartiles of the valid pixel distribution.  Input: hyperspectral cube only.

### DT Fedrop

Classifies iron drop intensity into eight classes; fedrop = R(1600 nm) / R(1310 nm).  Inputs: cube + WoM-C.

### DT Illcryst

Classifies illite crystallinity into eight classes from smectite to muscovite; illx = depth(2100–2400) / depth(1850–2100).  Inputs: WoM-B + WoM-C.

### DT Ill-Kaol

Classifies illite-to-kaolinite ratio into nine classes.  Inputs: WoM-B + WoM-C.

### DT Kaol-Cryst Classification

Classifies kaolinite pixels by crystallinity order using the 2162/2206 nm depth ratio (Crystallinity Index). Well-crystallised kaolinite shows a strong 2162 nm feature; poorly crystallised kaolinite and mixtures show a weaker or absent feature.  Inputs: FE05-2160D WoM raster + FE12-2206W WoM raster.

### DT Dick-Kaol Classification

Classifies Al-OH pixels into dickite or kaolinite using the wavelength position of the 2162/2178 nm shoulder feature. Dickite shoulder lies at ~2178 nm vs kaolinite ~2162 nm — a diagnostic 16 nm shift that persists in mixed pixels.  Inputs: FE12-2206W WoM raster + WoM from 2138–2196 nm range.

### DT Kandite Group Classification

Classifies Al-OH pixels into all five kandite group minerals: kaolinite, dickite, nacrite, halloysite, and metahalloysite. Uses the primary Al-OH wavelength, the 2162/2178 nm shoulder, and the 1900 nm water band depth.  Inputs: FE12-2206W WoM + WoM 2138–2196 nm + FE16-1900W WoM.

### DT Pyrophyllite Classification

Classifies Al-OH pixels for pyrophyllite using the diagnostic 2066 nm doublet and the 1396 nm OH overtone feature from the USGS GMEX pyrophyllite reference spectrum.  Inputs: FE17-2066D WoM raster + WoM from 2155–2180 nm range.

### DT Mineral Map

Classifies pixels into 17 mineral classes using W1, W2, and Ix.  Inputs: WoM-B + WoM-C (WoM-C must have ≥2 features).

Each classifier produces a palette-indexed integer class raster (GeoTIFF) with class names in a Raster Attribute Table (RAT). An optional RGB colour raster can also be requested.

# 9. SAM Classification

The Spectral Angle Mapper classifies image pixels by measuring the spectral angle between each pixel and a set of reference spectra from a spectral library. SAM is insensitive to illumination variations, making it robust for mineral identification.

## Spectra Filter

| Filter Pattern | Effect |
| --- | --- |
| Kaolinite* | All spectra starting with Kaolinite |
| *alunite* | Any spectrum containing alunite (case-insensitive) |
| Kaolinite*,Alunite* | Multiple mineral groups |
| 1-10 | First 10 spectra by index |
| 1,5,42 | Specific spectra by index |
| (empty) | Use all spectra in the library |

## Similarity Measures

| Code | Measure | Notes |
| --- | --- | --- |
| SAM | Spectral Angle Mapper | Recommended. Insensitive to illumination. |
| BC | Bray-Curtis Distance | Compositional dissimilarity. Range [0,1]. |
| SID | Spectral Information Divergence | Information-theoretic. Sensitive to spectral shape. |
| ED | Euclidean Distance | Sensitive to shape and magnitude. |
| ID | Intensity Difference | Sum of absolute differences. |

# 10. Spectral Processing Tools

The spectral processing tools can be applied independently to any hyperspectral raster. All tools read wavelength metadata from the ENVI domain first, with fallback to the default metadata domain and numeric band descriptions.

## 10.1  Convex Hull Removal

Removes the spectral continuum from each pixel spectrum using a convex hull fit, isolating individual absorption features for analysis.

| Parameter | Description |
| --- | --- |
| Mode | Divide: spectrum / hull (absorptions < 1.0) ¦ Subtract: 1 + (spectrum – hull) |
| Cutoff Wavelength (nm) | Restrict to bands below this wavelength. Set to 0 to use all bands. |
| Use Bad Band List | Exclude bands flagged in ENVI BBL metadata |

## 10.2  Band Ratio

Computes a single-band ratio image: Numerator band / Denominator band, specified by wavelength in nm. The nearest available band is selected automatically.

## 10.3  Band Ratios (Sequential)

Computes sequential band ratios: output[i] = input[i] / input[i + delta]. Useful for highlighting spectral gradients and transitions.

## 10.4  Band Depths

Computes per-band absorption depths: top = (band[i−delta] + band[i+delta]) / 2, depth = (top − band[i]) / top. Positive values indicate absorption.

## 10.5  Band Math

Evaluates an arbitrary Python/NumPy expression over one or more input rasters. Input images are referenced as i1, i2, ...

| Syntax | Returns | Description |
| --- | --- | --- |
| i1(2200.0) | 2D array (rows × cols) | Band nearest to 2200 nm — wavelength call |
| i1[n] | 2D array (rows × cols) | Band at integer index n (0-based) |
| i1[j, i] | 1D array (bands,) | Spectrum at row j, col i |
| i1[...] | 3D array | Entire image cube |
| i1.wavelength | 1D array or None | Wavelength array from metadata |
| i1.bands | int | Number of (valid) bands |

| Expression | Description |
| --- | --- |
| i1(2200.0) / i1(2100.0) | Wavelength ratio |
| (i1(2200.0) - i1(2100.0)) / (i1(2200.0) + i1(2100.0)) | NDVI-style normalised difference |
| log(i1(2200.0)) - log(i1(2100.0)) | Log ratio |
| i1(2200.0) - i2(2200.0) | Difference between two images at same wavelength |

## 10.6  Spectra Math

Applies a per-pixel spectral expression using one or more input images. Each pixel spectrum is wrapped in a full HyPy Spectrum object (S1, S2, ...).

| Syntax | Returns | Description |
| --- | --- | --- |
| S1[2200.0] | scalar | Value at nearest wavelength to 2200 nm |
| S1(2200.0) | scalar | Same — callable syntax |
| S1[2000.0:2400.0] | Spectrum | Sub-spectrum 2000–2400 nm |
| S1[5] | scalar | Value at integer band index 5 |
| S1.spectrum | 1D array | Raw reflectance values |
| S1.wavelength | 1D array | Wavelength array |

When two Spectrum objects are combined with +, -, *, /, etc., they are automatically resampled onto their overlapping wavelength range — no manual resampling needed.

Available methods: S1.log()  S1.exp()  S1.sqrt()  S1.mean()  S1.min()  S1.max()  S1.clip(a,b)  S1.hull()  S1.nohull()  S1.peaks()  S1.smooth(n)  S1.gradient()  S1.localmin(n)  S1.minwav(n)  and more.

## 10.7  Log Residuals

Computes Log Residuals or Kwik Residuals normalisation — a standard technique for removing scene-wide albedo effects and sensor response variations from hyperspectral data. Optionally outputs an albedo band and/or a residual lower-upper boundary (RLUB) raster.

## 10.8  PCA — Forward

Performs forward Principal Components Analysis on a hyperspectral image using SVD decomposition. Outputs a multi-band PC score image (float32, BSQ) and a binary statistics file (.pca_stats) containing the mean vector, singular values, and eigenvector matrix required by a future Inverse PCA tool.

| Parameter | Description |
| --- | --- |
| Number of PC components | How many PCs to output (0 = all bands). For most mineralogical applications, 10–20 components capture virtually all spectral variance. |
| Sort bands by wavelength | Reorders bands before analysis. |
| Exclude bad bands | Uses the Bad Band List from the ENVI header. |
| Restrict to bounding box | If ticked, the covariance matrix and eigenvectors are computed from the specified sub-region only. The full image is still projected into the derived PC space. |
| NaN-safe mode | Removes pixels with any NaN band before computing statistics. Required for masked or partially-nodata images. |
| Output Stats File (.pca_stats) | Binary pickle file with mean vector, singular values, and eigenvectors. Required for Inverse PCA (future tool). |

## 10.9  Linear Filter (3×3 Convolution)

Applies a 3×3 spatial convolution kernel independently to each band of a hyperspectral image:

output = bias × (kernel ⊛ input) + offset

The 1-pixel border of each output band is set to zero. Four built-in presets plus a fully user-definable custom kernel.

| Preset | Kernel | Use |
| --- | --- | --- |
| Smoothing | Uniform 3×3 box (all weights = 1) | General spatial smoothing |
| Laplacian | 0 -1 0 / -1 4 -1 / 0 -1 0 | Edge-enhance; highlight spectral gradients |
| Sharpen | 0 -1 0 / -1 5 -1 / 0 -1 0 | Centre-weighted spatial sharpening |
| Emboss | Directional weighted | Directional emboss effect |
| Custom | User-defined (3 rows of 3 values) | Any arbitrary 3×3 kernel |

## 10.10  3D Median / Mean Filter

Applies a 3D spatial-spectral median or mean filter to a hyperspectral image. The filter operates simultaneously across spatial (x, y) and spectral (band) dimensions, removing random 3D noise such as detector cosmetic blemishes or isolated bad pixels, while preserving spectral shape better than a purely spatial filter.

NaN values are ignored in all modes (nanmedian / nanmean). The 1-pixel spatial border and 1-band spectral border are copied unchanged from the input.

| Mode | Neighbourhood | Notes |
| --- | --- | --- |
| fmed7 / mean7 | 7-point cross (centre + 6 face-adjacent) | Fastest; minimal spatial-spectral mixing |
| fmed19 / mean19 | 19-point cross (face + edge-adjacent) | Balanced noise removal |
| fmed27 / mean27 | Full 3×3×3 cube (27 points) | Most thorough denoising; also most mixing |

Memory Note: the entire image cube must be held in memory during 3D filtering. For very large images, run a Subset first to reduce scene size.

## 10.11  Zonal Statistics

For each unique zone in a single-band zones raster, computes a per-band spectral statistic across all pixels of the hyperspectral image that fall within that zone, and writes one tab-separated text file per zone. Output columns are wavelength (nm) and the chosen statistic — ready for plotting as a representative spectrum.

Typical workflow: classify a scene with a Decision Tree or SAM classifier, then supply the classification output as the zones raster to extract a mean (or percentile) spectrum per mineral class.

| Parameter | Description |
| --- | --- |
| Zones raster | Single-band integer raster. ENVI Classification files carry class_names used in output filenames. Must match the spatial dimensions of the input image. |
| Input image | Hyperspectral raster (.hdr or binary data file). |
| Method | mean (default), median, max, min, std, sum, m-2s (mean − 2·std), m+2s (mean + 2·std). |
| Use Bad Band List | Skip bands flagged in the ENVI BBL. |
| Sort by wavelength | Reorder output rows by ascending wavelength. |
| Output folder | Where to write the .txt files. Leave blank to write beside the input image. |

Output filenames follow the pattern <image>_<zone>_<method>.txt — e.g. f250610_Kaolinite_mean.txt. Each file is a tab-separated table of wavelength (nm) vs statistic value, with a commented header block recording all run parameters.

# 11. Spectrum Viewer

The Spectrum Viewer is a dockable panel for interactive pixel spectrum inspection and comparison against a spectral library. Open via HypPy → Spectrum Viewer from the toolbar dropdown.

- Pick a pixel: Click ▶ Pick pixel to activate the crosshair cursor, then click any pixel on the map canvas to display its spectrum. Right-click or press Esc to stop.

- Plot library spectra: Select a Library type, browse to your library file, select one or more spectra, and click Plot to overlay them on the right Y axis.

- Math Expression: Enter a spectral expression (e.g. S.nohull(), S[2200.0], S.mean()) and click Go to evaluate against the current pixel spectrum.

- View Values: Switch to the Values tab for a wavelength / reflectance table of all plotted spectra.

- Export: Right-click the plot area to copy the pixel spectrum as CSV or save the plot as an image.

Note: Negative reflectance values (common in noisy detector regions, typically beyond ~2400 nm) are masked and displayed as gaps in the plot.

# 12. Convert Tools

The Convert Tools group provides format conversion utilities, available via the Processing Toolbox under 8 - Convert Tools and via the HypPy toolbar dropdown.

| Tool | Description |
| --- | --- |
| G01 — Convert Image to ENVI | Converts TIFF, JPEG, PNG, or BMP to ENVI flat-binary format. Georeferencing is preserved from GeoTIFF tags if present. |
| G02 — Convert EOS HDF to ENVI | Extracts every EOS_SWATH subdataset from an EOS HDF file and writes a separate ENVI file and .hdr per band. Requires GDAL with HDF4 support. |
| G03 — Convert ASTER HDF to ENVI | Extracts ImageData/Band/Temperature subdatasets from an ASTER HDF file. Locates GDAL executables automatically from the QGIS/OSGeo4W environment on Windows. |
| G04 — Convert EMIT L2A to ENVI | Reads an EMIT L2A NetCDF4 reflectance file and writes four ENVI outputs (_refl, _lonlat, _elev, _glt). Requires the netCDF4 Python package. |

# 13. Common Wavelength Ranges

| Application | Start (nm) | End (nm) | Target Features |
| --- | --- | --- | --- |
| Clay minerals (AlOH) | 2100 | 2250 | Kaolinite ~2165, Montmorillonite ~2210 |
| Broad SWIR survey | 2000 | 2500 | General mineral mapping |
| Carbonate minerals | 2300 | 2400 | Calcite ~2340, Dolomite ~2320 |
| Iron oxides | 850 | 1050 | Hematite ~860, Goethite ~920 |
| Vegetation red edge | 680 | 750 | Chlorophyll absorption |
| Vegetation water | 900 | 1100 | Leaf water content ~970 nm |

# 14. Version History

| Ver. | Title | Key Changes |
| --- | --- | --- |
| 2.45 | PCA, Linear Filter, 3D Filter + Workflow Guide update | New Spectral Processing Tools: PCA — Forward (SVD-based PCA with optional bounding-box stats and NaN-safe mode, outputs .pca_stats for future Inverse PCA); Linear Filter (3×3 spatial convolution with Smoothing, Laplacian, Sharpen, Emboss and Custom presets); 3D Median / Mean Filter (3D spatial-spectral denoising in fmed7/19/27 and mean7/19/27 modes, NaN-safe). Workflow Guide (Step 1) updated to reflect 8-group structure, all DT classifiers, and pre-processing guidance. |
| 2.44 | Subset race-condition fix | Fixed Windows fatal exception (access violation) crash when using D02 Subset with "Use map coordinates" + "Use layer extent". Cause: parameterAsExtent() was called after gdal.Open(), triggering a QGIS background QgsTask that raced with ReadAsArray(). Fix: resolve all parameters — including extent — before opening the GDAL dataset. |
| 2.43 | New DT classifiers | Added DT Kaol-Cryst, DT Dick-Kaol, DT Kandite Group, and DT Pyrophyllite classifiers. Each uses FE raster outputs as inputs and targets specific kaolin-group and phyllosilicate minerals using USGS GMEX reference spectral parameters. |
| 2.38 | E09 Zonal Statistics | New Spectral Processing Tool: E09 - Zonal Statistics. For each zone in a single-band zones raster, writes a tab-separated .txt file of wavelength (nm) vs statistic for all pixels in that zone. Eight methods: mean, median, max, min, std, sum, m-2s, m+2s. ENVI class names used in filenames. Supports BBL and sort-by-wavelength. Ported from HyPy 3.0.1 zonalstatistics.py. |
| 2.37 | Data Import group | New toolbox group "2 - Data Import" inserted between Workflow Guide and Generate WoM Rasters. Build Overviews moved from Convert Tools (renamed D01). Subset moved from Spectral Processing Tools (renamed D02). All other groups renumbered 3–8. Toolbar and provider updated. |
| 2.36 | D01: auto-load + transparency | After building overviews, image automatically loaded into map canvas with -9999 values transparent. Canvas zooms to image extent on load. |
| 2.35 | D01: .hdr input fix | D01 resolves an ENVI .hdr path to the binary data file before opening with GDAL, fixing the "select the data file" error. |
| 2.33 | Build Overviews | New tool: builds external GDAL overview pyramids (.ovr sidecar). Three level presets, five resampling methods, progress bar. |
| 2.55 | FE ranges aligned to L21 Table 5 | WoM search ranges and map colour stretch values aligned to Laukamp et al. (2021) Table 5 (SWIR continuum removal ranges). FE01–FE03 WoM widened to 1350–1450 nm. FE08/FE11 ranges corrected. FE15 stretch corrected to 2279–2338 nm. FE07/FE08 references updated to GMEX. Reference column updated to cite Table 5. |
| 2.54 | FE table Reference column | Added Reference column to FE table (§7.1) linking each tool to Laukamp et al. (2021) or Turner et al. (2014/2016/2018). Added Turner et al. REE references to §15. |
| 2.53 | Laukamp et al. 2021 reference | Added Laukamp, C. et al., 2021 (Minerals 11(4), 347) to References in USER_GUIDE.md, README.md, getting_started help, and dt_mineral_map help. |
| 2.32 | HyPy journal reference | Added Bakker et al., 2024 (Algorithms 17(8), Article 337) to References. |
| 2.23 | Spectrum Viewer stability | Fixed RuntimeError when picking pixels after dock widget has been closed or plugin reloaded. |
| 2.21 | Negative reflectance masking | Pixel spectrum values below zero masked to NaN and shown as gaps in Spectrum Viewer. |
| 2.11 | Full HyPy Spectrum class | Spectra Math: float wavelength indices, auto-resampling, full numpy method set. Band Math: wavelength __call__ syntax. |
| 2.10 | Wavelength unit normalisation | All algorithms auto-convert nm / µm / wavenumber. |
| 2.9 | Log Residuals, SAM improvements | New Log Residuals tool. SAM vectorised (10–50× speedup), UInt16 output, USGS splib07 parser fix. Polynomial fit order option for WoM-B. |
| 2.8 | Feature Extraction group | Added 7 - Feature Extraction with FE01–FE11 one-click tools. |
| 2.4 | Toolbox restructure | Reorganised into current group scheme. Fixed dt_2100_2400 mineral class bug. |
| 2.3 | Resizable legend window | Legend dock fully resizable by dragging its edges. |
| 2.0 | Legend floating dock | Legend displayed automatically in a floating QDockWidget. |
| 1.8 | Plugin rename | Plugin renamed to HypPy_for_QGIS. |
| 1.4 | Workflow A & dt_mineral_map | New Workflow A generates WoM rasters directly. New 17-class mineral map classifier. |
| 1.1 | Spectral Processing Tools | Added Convex Hull Removal, Band Ratio, Band Ratios, Band Depths, Band Math, Spectra Math. |

# 15. References

Kruse, F.A. et al., 1993. The Spectral Image Processing System (SIPS). Remote Sensing of Environment, 44(2-3), pp.145–163.

Kokaly, R.F. et al., 2017. USGS Spectral Library Version 7. U.S. Geological Survey Data Series 1035, 61 p.

Bakker, W.H., 2010–2022. HyPy — Hyperspectral Python. University of Twente, Faculty ITC.

Bakker, W. et al., 2024. Hyperspectral Python: HypPy. Algorithms, 17(8), Article 337. https://doi.org/10.3390/a17080337

van Ruitenbeek, F.J.A. et al., 2025. A Knowledge-Based Strategy for Interpretation of SWIR Hyperspectral Images of Rocks. Remote Sensing, 17(15), 2555. https://doi.org/10.3390/rs17152555

Laukamp, C., Rodger, A., LeGras, M., Lampinen, H., Lau, I.C., Pejcic, B., Stromberg, J., Francis, N. and Ramanaidou, E., 2021. Mineral Physicochemistry Underlying Feature-Based Extraction of Mineral Abundance and Composition from Shortwave, Mid and Thermal Infrared Reflectance Spectra. Minerals, 11(4), 347. https://doi.org/10.3390/min11040347

Turner, D.J., Rivard, B. and Groat, L.A., 2014. Visible and Short-Wave Infrared Reflectance Spectroscopy of REE Fluorocarbonates. American Mineralogist, 99(7), pp.1335–1346.

Turner, D.J., Rivard, B. and Groat, L.A., 2016. Visible and Short-Wave Infrared Reflectance Spectroscopy of REE Phosphate Minerals. American Mineralogist, 101(10), pp.2264–2278.

Turner, D.J., Rivard, B. and Groat, L.A., 2018. Visible and Short-Wave Infrared Reflectance Spectroscopy of Selected REE-Bearing Silicate Minerals. American Mineralogist, 103(6), pp.927–943.