Hyppy for QGIS

Complete Workflow Guide

Hyperspectral Analysis Tools  |  Version 2.19  |  March 2026

Grant Boxer

boxerg@iinet.net.au

Original HyPy algorithms: Wim Bakker, University of Twente

# Table of Contents

# 1. Overview

The Hyppy for QGIS plugin provides hyperspectral analysis tools accessed via the QGIS Processing Toolbox. The toolbox is organised into six numbered groups:

- 1 - Workflow Guide — open this guide in your browser

- 2 - Generate WoM Rasters — generate Wavelength of Minimum rasters and wavelength maps

- 3 - Decision Tree Classification — classify minerals and physical properties

- 4 - SAM Classification — Spectral Angle Mapper classification against a spectral library

- 5 - Spectral Processing Tools — band ratios, depths, convex hull removal, log residuals, band math

- 6 - Feature Extraction — one-click WoM + wavelength map + legend for standard absorption features

## 1.1 Algorithm Summary

| Algorithm | Group | Purpose |
| --- | --- | --- |

Step 1 - Open Workflow Guide1 - Workflow GuideOpen this guide in a browserStep 2a - Generate All Three WoM Rasters2 - Generate WoM RastersGenerate WoM-A, WoM-B, WoM-C in one stepStep 2a - Generate WoM-A (750-1300 nm)2 - Generate WoM RastersGenerate WoM-A individuallyStep 2a - Generate WoM-C (2100-2400 nm)2 - Generate WoM RastersGenerate WoM-C individuallyStep 2b - Generate WoM-B (1850-2100 nm)2 - Generate WoM RastersGenerate WoM-B with choice of polynomial fit orderStep 2b - Wavelength of Minimum (custom range)2 - Generate WoM RastersExtract features for any custom wavelength rangeStep 2c - Wavelength Mapping2 - Generate WoM RastersCreate RGB visualisation from absorption featuresFE01-FE18 — Feature Extraction6 - Feature ExtractionOne-click WoM + map + legend per featureDT Mineral 2100-24003 - Decision Tree ClassificationClassify 16 mineral classes from WoM-CDT Albedo3 - Decision Tree ClassificationClassify brightness from mean reflectanceDT Fedrop3 - Decision Tree ClassificationClassify iron drop from R(1600)/R(1310)DT Illcryst3 - Decision Tree ClassificationClassify illite crystallinity and smectiteDT Ill-Kaol3 - Decision Tree ClassificationClassify illite vs kaolinite ratioDT Mineral Map3 - Decision Tree ClassificationClassify 17 mineral classes from WoM-B + WoM-CSpectral Angle Mapper (SAM)4 - SAM ClassificationClassify pixels against a spectral libraryConvex Hull Removal5 - Spectral Processing ToolsContinuum removal using convex hull fittingBand Ratio5 - Spectral Processing ToolsSingle-band ratio by wavelengthBand Ratios (Sequential)5 - Spectral Processing ToolsAll-band sequential ratios with deltaBand Depths5 - Spectral Processing ToolsPer-band absorption feature depthsBand Math5 - Spectral Processing ToolsNumPy expression over image bands (i1, i2...)Spectra Math5 - Spectral Processing ToolsPer-pixel spectral expression (S1, S2...)Log Residuals5 - Spectral Processing ToolsLog Residuals / Kwik Residuals normalisation

# 2. Installation

Extract the Hyppy_for_QGIS_v2_9 folder from the ZIP and copy it into your QGIS plugins directory:

- Windows: %APPDATA%\QGIS\QGIS3\profiles\<profile>\python\plugins\

- Linux: ~/.local/share/QGIS/QGIS3/profiles/<profile>/python/plugins/

- macOS: ~/Library/Application Support/QGIS/QGIS3/profiles/<profile>/python/plugins/

Restart QGIS, then enable the plugin via Plugins > Manage and Install Plugins. The Hyppy tools will appear in the Processing Toolbox under the Hyppy provider.

## 2.1 Requirements

- QGIS 3.16 or later

- Python 3.x with NumPy and SciPy (included with QGIS)

- GDAL/OGR (included with QGIS)

## 2.2 Installation from ZIP

QGIS can install plugins directly from a ZIP file via Plugins > Manage and Install Plugins > Install from ZIP. Browse to Hyppy_for_QGIS_v2_9.zip and click Install. If the ZIP file produces a BadZipFile error, the file was corrupted during transfer — re-download or share via Google Drive rather than email.

# 3. Step 2 — Generate WoM Rasters

This group generates the three Wavelength of Minimum (WoM) rasters required by the decision tree classifiers, and provides tools for custom wavelength range analysis and wavelength mapping. Run the WoM generation step first whenever processing a new hyperspectral dataset.

Use Step 2a - Generate All Three WoM Rasters to produce WoM-A, WoM-B, and WoM-C in a single operation, or run the individual Step 2a / Step 2b algorithms if only specific ranges need to be regenerated.

## 3.1 Required Input

The hyperspectral spectral cube — the original reflectance image with full spectral range and wavelength metadata (ENVI format or equivalent). All Step 2 algorithms accept the cube as their primary input.

## 3.2 WoM Raster Specifications

| Name | Wavelength Range | Used by | Notes |
| --- | --- | --- | --- |

WoM-A750–1300 nmFuture classifiersIron oxide and hydroxyl featuresWoM-B1850–2100 nmdt_illcryst, dt_ill_kaol, dt_mineral_mapWater/carbonate shoulder; illite crystallinity denominatorWoM-C2100–2400 nmdt_mineral_map, dt_2100_2400, dt_fedrop, dt_illcryst, dt_ill_kaolPrimary clay mineral absorption region

## 3.3 Step 2a Algorithms

### Generate All Three WoM Rasters

Runs the Wavelength of Minimum algorithm three times in sequence on a single hyperspectral cube, producing WoM-A, WoM-B, and WoM-C in one operation. This is the recommended starting point for any new dataset.

### Generate WoM-A / WoM-C (individual)

Individual algorithms for regenerating a single WoM raster without reprocessing the others. Pre-configured with the correct wavelength range; the only required inputs are the cube and an output path. Use Division continuum removal (default). Default 3 features per pixel (1–9 selectable).

## 3.4 Step 2b — Generate WoM-B (1850–2100 nm)

Generates the WoM-B raster for the 1850–2100 nm range. In addition to the standard number-of-features setting, Step 2b exposes a Polynomial fit order parameter that controls how precisely the sub-pixel absorption minimum is located.

| Parameter | Options | Notes |
| --- | --- | --- |

Hyperspectral cube—Input reflectance image with wavelength metadataNumber of features (1–9)1–9 (default 3)How many absorption minima to extract per pixelPolynomial fit order2nd order (default) | 4th orderSee belowOutput—WoM-B GeoTIFF

### Polynomial Fit Order

After convex hull continuum removal, the Wavelength of Minimum algorithm locates local minima to band-level precision, then fits a polynomial curve through the surrounding points to refine the minimum to sub-band precision.

- 2nd order (quadratic) — default. Fits a parabola through the 3 points centred on each minimum and finds the vertex analytically. Fast and reliable for narrow, symmetric absorption features. Sufficient for most clay mineral mapping workflows.

- 4th order (quartic). Fits a quartic polynomial through the 5 points centred on each minimum. The minimum of the fitted curve is located via the real roots of its derivative. Better suited to asymmetric or broad features where the parabola does not adequately capture the true minimum shape. Typical improvement on an asymmetric absorption: ~1 nm additional accuracy.

💡 The polynomial fit order is also available in the full Step 2b — Wavelength of Minimum (custom range) tool, so it applies to any wavelength window, not just 1850–2100 nm.

## 3.5 Step 2b — Wavelength of Minimum (Custom Range)

Analyses each pixel's spectrum within a user-defined wavelength range. Use this when you need a WoM raster for a wavelength range other than the three presets in Step 2a.

| Parameter | Typical Value | Notes |
| --- | --- | --- |

Input ImageHyperspectral rasterMust have wavelength metadataStart Wavelength (nm)2000 (0 = first band)Begin analysis windowEnd Wavelength (nm)2500 (0 = last band)End analysis windowContinuum RemovalDivisionDivision recommended for mineralsNumber of features (1–9)1–31 for single mineral, 3+ for multi-mineral mappingBroad feature fittingOffOn for very broad absorption featuresPolynomial fit order2nd orderSee Section 3.4 aboveUse Bad Band ListOnExcludes noisy bands flagged in ENVI metadata

Output: Multi-band GeoTIFF containing wavelength/depth pairs for each requested feature. Band 1 = interpolated minimum wavelength (nm), Band 2 = interpolated depth. Features are sorted deepest first.

## 3.6 Step 2c — Wavelength Mapping

Takes a WoM output raster and creates an RGB image. Colour (hue) represents absorption wavelength position and brightness represents absorption depth.

| Parameter | Typical Value | Notes |
| --- | --- | --- |

Input (Step 2a or 2b output)WoM GeoTIFFUses Band 1 (wavelength) and Band 2 (depth)Min Wavelength for Display (nm)2100Blue end of colour stretchMax Wavelength for Display (nm)2350Red end of colour stretchMin Depth for Display0.0Lower bound of brightness stretchMax Depth for Display0.2Adjust for data range (try 0.05–0.3)

Output Wavelength Map (RGB): 3-band GeoTIFF. Hue encodes wavelength position across the stretch range; brightness encodes absorption depth.

Output Wavelength Map Legend (PNG): 2D colour legend. Y axis = wavelength (nm), X axis = absorption depth. After processing, the legend is automatically displayed in a floating dock window inside QGIS. The dock is resizable by dragging its edges.

# 4. Feature Extraction (Group 6)

The Feature Extraction tools are one-click pipelines that combine Wavelength of Minimum, Wavelength Mapping, and legend display into a single operation. Each tool targets a specific known absorption feature with pre-configured wavelength ranges and map stretch values.

Running a Feature Extraction tool produces three outputs: a WoM raster, an RGB wavelength map, and a colour legend PNG — and automatically displays the legend in a floating dock window after processing.

## 4.1 Feature Extraction Tools

| Tool | WoM Range | Map Colour Stretch | Characteristic Feature |
| --- | --- | --- | --- |

FE1 - 1480W1440–1520 nm1471–1491 nmWater / hydroxyl combination ~1480 nmFE2 - 1550W1510–1610 nm1520–1563 nmHydroxyl overtone / organic ~1550 nmFE3 - 1760W1730–1790 nm1751–1764 nmCarbonate / organic combination ~1760 nmFE4 - 2080D2060–2100 nm2075–2085 nmCarbonate / AlOH diagnostic ~2080 nmFE5 - 2160D2138–2179 nm2159–2166 nmKaolinite / dickite diagnostic ~2160 nmFE6 - 2200W2120–2245 nm2185–2215 nmAlOH absorption — illite / smectite / kaoliniteFE7 - 2250W2230–2280 nm2248–2268 nmAlOH / MgOH transition ~2250 nmFE8 - 2290W2270–2320 nm2279–2338 nmMgOH / carbonate ~2290 nmFE9 - 2320W2295–2345 nm2300–2340 nmCarbonate / dolomite ~2320 nmFE10 - 2350W2310–2370 nm2320–2366 nmCarbonate / chlorite ~2350 nmFE11 - 2390W2375–2435 nm2377–2406 nmMgOH / chlorite / serpentine ~2390 nmFE12 - 2206W2162–2245 nm2197–2215 nmKaolinite primary Al-OH ~2206 nm (GMEX kaolinite2)FE13 - 1400D1385–1430 nm1395–1420 nmKaolinite OH doublet ~1400/1412 nm (GMEX kaolinite2). Satellite data only — BBL-masked in airborne.FE14 - 2178D2155–2196 nm2168–2190 nmDickite Al-OH shoulder ~2178 nm (+16 nm from kaolinite ~2162 nm). Persists in mixed spectra (GMEX dickite).FE15 - 1384D1370–1435 nm1378–1425 nmDickite OH doublet ~1384/1418 nm. Wider separation than kaolinite ~1400/1412 nm (GMEX dickite). Satellite data only.FE16 - 1900W1850–1960 nm1870–1940 nmHalloysite interlayer water band ~1900 nm. Primary discriminator separating halloysite from all dry kandite minerals (GMEX kandite group).FE17 - 2066D2050–2100 nm2060–2090 nmPyrophyllite Al-OH doublet ~2066/2078 nm; “may persist in mixtures” (GMEX pyrophyllite). Default 2 features. Use with FE18 and DT Pyrophyllite.FE18 - 1396W1380–1420 nm1388–1406 nmPyrophyllite single sharp OH ~1396 nm (GMEX). Single peak vs kaolinite doublet ~1400/1412 nm and dickite doublet ~1384/1418 nm. Satellite data only — BBL-masked in airborne.

W suffix = water or hydroxyl combination band.  D suffix = diagnostic mineral absorption feature.

## 4.2 Inputs

| Parameter | Description |
| --- | --- |

Hyperspectral cubeThe original reflectance image with wavelength metadataNumber of features (1–9)How many absorption minima to extract per pixel (default 1)Output WoM rasterPath for the Wavelength of Minimum outputOutput Wavelength Map RGBPath for the colour-coded mapOutput Colour Legend (PNG)Optional — defaults to <label>_legend.png beside the WoM raster

## 4.3 Workflow Tip

Feature Extraction tools are designed for rapid reconnaissance of a dataset. Run several tools across your scene to identify which absorption features are present and spatially coherent before committing to a full decision tree classification workflow.

# 5. Decision Tree Classification (Group 3)

The decision tree classifiers use pre-computed wavelength analysis outputs to assign mineral or physical property classes to each pixel. All classifiers require a minimum set of input data prepared in Step 2.

## 5.1 Required Input Data

### Hyperspectral spectral cube

Required by dt_albedo and dt_fedrop to compute internal band ratios.

### WoM rasters from Step 2

| WoM | Wavelength Range | Used by |
| --- | --- | --- |

WoM-A750–1300 nmFuture classifiersWoM-B1850–2100 nmdt_mineral_map, dt_illcryst, dt_ill_kaol (depth denominator)WoM-C2100–2400 nmdt_mineral_map, dt_2100_2400, dt_fedrop, dt_illcryst, dt_ill_kaol

## 5.2 Decision Tree Algorithms

### DT Mineral 2100-2400 — Mineral Classification (2100–2400 nm)

Classifies 16 mineral classes including kaolinite, muscovite, illite, pyrophyllite, alunite, and carbonate minerals. Input: WoM-C only. Uses the primary absorption wavelength (Band 1) and feature depth (Band 2).

Decision tree after van Ruitenbeek et al. (2025), Remote Sensing 17(15), 2555.

### DT Albedo — Brightness Classification

Classifies pixels into four brightness classes (dark, med-dark, med-light, light) based on mean reflectance. Thresholds are calculated dynamically using quartiles of the valid pixel distribution. Input: hyperspectral cube only.

### DT Fedrop — Iron Drop Classification

Classifies iron drop intensity into eight classes (aspectral, no2100-2400, low through very-high). The fedrop value R(1600 nm) / R(1310 nm) is computed internally. Inputs: hyperspectral cube + WoM-C.

### DT Illcryst — Illite Crystallinity Classification

Classifies illite crystallinity into eight classes ranging from smectite to muscovite. The illite crystallinity value is computed internally as depth(2100–2400 nm) / depth(1850–2100 nm). Inputs: WoM-B + WoM-C.

### DT Ill-Kaol — Illite / Kaolinite Classification

Classifies the illite-to-kaolinite ratio into nine classes (aspectral, NO-ALOH, kaol4–kaol1, ill1–ill4). Inputs: WoM-B + WoM-C.

### DT Mineral Map — Mineral Map Classification

Classifies pixels into 17 mineral classes covering illite/muscovite variants, kaolinite, phengite, Fe-chlorite, and chlorite. Branches on W1 (wavelength of deepest feature), W2 (second deepest), and Ix (illite crystallinity). Inputs: WoM-B + WoM-C (WoM-C must have ≥2 features).

### DT Kaol-Cryst — Kaolinite Crystallinity Classification

Classifies kaolinite pixels into four crystallinity classes (kaol_hi, kaol_med, kaol_lo, kaol_dis) using a Crystallinity Index (CI = depth(2162) / depth(2206)). Higher CI indicates a better-ordered crystalline structure. Inputs: FE5-2160D + FE12-2206W. Spectral parameters from AusSpec International (2005) GMEX kaolinite2 reference spectrum.

### DT Dick-Kaol — Dickite / Kaolinite Classification

Discriminates dickite from kaolinite using the Al-OH shoulder wavelength position. Dickite has its shoulder at ~2178 nm, shifted +16 nm from kaolinite’s ~2162 nm — a diagnostic difference that persists in mixed spectra (AusSpec International 2005, GMEX dickite). Four output classes: kaolinite (≤2167 nm), kaol_mixed, dick_mixed, dickite (>2183 nm). Inputs: FE12-2206W + custom WoM over 2138–2196 nm.

### DT Kandite — Kandite Group Classification

Full five-member kaolin (kandite) group classifier derived from the GMEX kandite group comparison spectra (AusSpec International 2005). Classifies Al-OH pixels into kaolinite, dickite, halloysite (hydrated), halloysite (dehydrated), and nacrite using three spectral parameters in strict priority order: depth(2206) gate; wavelength gate (2197–2215 nm); depth(1900) for halloysite; shoulder wavelength for nacrite and dickite/kaolinite split. Inputs: FE12-2206W + custom WoM 2138–2196 nm + FE16-1900W. Processing log reports water band depth statistics and percentage halloysite of kandite pixels to assist threshold review.

### DT Pyrophyllite — Pyrophyllite Classification

Classifies Al-OH pixels for pyrophyllite (Al₄[Si₈O₂₀](OH)₄) using the diagnostic spectral features from the USGS GMEX pyrophyllite reference spectrum (AusSpec International 2005). Primary gate: depth(2166) and wavelength position 2158–2175 nm. Secondary confirmation: depth of the ~2066/2078 nm doublet (GMEX: “may persist in mixtures”). Output classes route non-pyrophyllite Al-OH to possible_illite (2175–2200 nm) or possible_kandite (2200–2220 nm). Confirmed pyrophyllite is further split by doublet position into pyro_doublet_lo, pyrophyllite (centred), and pyro_doublet_hi. Inputs: custom WoM 2155–2180 nm + FE17-2066D. Processing log reports doublet depth and primary Al-OH wavelength statistics.

## 5.3 Outputs

Each classifier produces a palette-indexed integer class raster (GeoTIFF). Class names are embedded as a Raster Attribute Table (RAT), which QGIS reads to display class names in the layer legend. An optional RGB colour raster can also be requested for visual display.

# 6. Spectral Processing Tools (Group 5)

Seven tools that can be applied independently to any hyperspectral raster. All tools read wavelength metadata from the ENVI domain first, with fallback to the default metadata domain and numeric band descriptions.

## 6.1 Convex Hull Removal (Continuum Removal)

Removes the spectral continuum from each pixel spectrum using a convex hull fit, isolating individual absorption features for analysis.

| Parameter | Description |
| --- | --- |

Input ImageHyperspectral raster with wavelength metadataModeDivide: spectrum / hull (absorptions < 1.0)  |  Subtract: 1 + (spectrum − hull)Cutoff Wavelength (nm)Restrict to bands below this wavelength. Set to 0 for all bands.Use Bad Band ListExclude bands flagged in ENVI BBL metadata

## 6.2 Band Ratio

Computes a single-band ratio image: Numerator band / Denominator band. Bands are specified by wavelength in nm; the nearest available band is selected automatically.

## 6.3 Band Ratios (Sequential)

Computes sequential band ratios across the full spectrum: output[i] = input[i] / input[i + delta]. Useful for highlighting spectral gradients and transitions.

## 6.4 Band Depths

Computes per-band absorption feature depths. For each centre band i:

top = (band[i-delta] + band[i+delta]) / 2   depth = (top - band[i]) / top

Positive values indicate absorption.

## 6.5 Band Math

Evaluates an arbitrary Python/NumPy expression over one or more input rasters to produce a single-band output. Input images are referenced as i1, i2, ... (up to 20).

| Expression | Description |
| --- | --- |

i1.get_band(10) / i1.get_band(20)Band ratio (0-based indices, bands 11 and 21)(i1.get_band(30) - i1.get_band(20)) / (i1.get_band(30) + i1.get_band(20))Normalised difference (NDVI-style)i1.get_band(0) - i2.get_band(0)Band 1 difference between two images

## 6.6 Spectra Math

Applies a per-pixel spectral expression using one or more input images. Each pixel spectrum is wrapped in a Spectrum object (S1, S2, ... up to 20).

| Expression | Description |
| --- | --- |

S1 + S2Element-wise sum of two spectraSpectrum(S1.spectrum / S2.spectrum, S1.wavelength)Per-pixel spectral ratioSpectrum(np.log(S1.spectrum), S1.wavelength)Log of spectrum

## 6.7 Log Residuals

Normalises hyperspectral imagery using the Log Residuals or Kwik Residuals algorithm. This is a classic pre-processing step that removes albedo effects and broad spectral slope, highlighting subtle mineralogical absorption features.

### Algorithm

The 3-pass algorithm (Crosta & McMoore 1989; Bakker, University of Twente):

- Pass 1 — Albedo removal. Each pixel spectrum is divided by its geometric mean. The geometric mean represents the overall brightness (albedo) of the pixel, so dividing by it removes illumination variations while preserving spectral shape.

- Pass 2 — Robust Least Upper Bound (RLUB). For each band, the normalised image values are analysed in log space to compute a mean and standard deviation. The RLUB is set to geometric_mean + N×std_dev, capped at the 99th percentile. This estimates the spectral envelope due to broad absorptions and atmospheric effects.

- Pass 3 — RLUB division. Each normalised spectrum is divided by the RLUB, removing the broad spectral slope and leaving residual fine-scale absorption features.

### Parameters

| Parameter | Options | Notes |
| --- | --- | --- |

Input Image—Hyperspectral raster with wavelength metadataResiduals ModeLog Residuals (default) | Kwik ResidualsSee belowN Standard Deviations0.5–10 (default 3.0)Controls RLUB upper bound tightnessUse Bad Band ListOn / OffHonours ENVI BBL metadataOutput Log-Residuals Image—Primary output: normalised Float64 GeoTIFFOutput Albedo Image (optional)—Single-band raster of per-pixel geometric meanOutput RLUB Text File (optional)—Wavelength vs RLUB per band — for QC or plotting

### Log Residuals vs Kwik Residuals

- Log Residuals (recommended). Uses the geometric mean (computed in log space) for albedo removal and RLUB calculation. Theoretically rigorous and preferred for geological SWIR analysis.

- Kwik Residuals. Uses the arithmetic mean instead. Faster and simpler, but less theoretically sound. Can be useful as a quick check or when data values include zeros that prevent log computation.

💡 N Standard Deviations controls how tightly the RLUB fits the data. Lower values (e.g. 2.0) produce a tighter bound that may over-correct strong absorptions; higher values (e.g. 4.0) are more conservative. The default of 3.0 is appropriate for most mineral mapping applications.

# 7. SAM Classification (Group 4)

The Spectral Angle Mapper classifies image pixels by measuring the spectral angle between each pixel and a set of reference spectra. SAM is insensitive to illumination variations (albedo), making it robust for mineral identification from atmospherically corrected reflectance data.

## 7.1 Prepare Input Data

- Hyperspectral image with wavelength metadata in ENVI header format.

- Spectral library in ENVI binary format (.sli + .hdr) or CSV text format (.csv, .txt).

## 7.2 Browse the Spectral Library

To list available spectra without running SAM:

- Select your spectral library file

- Check List available spectra names (does not run SAM)

- Click Run — all spectra are listed with index numbers in the processing log

## 7.3 Spectra Filter

| Filter Pattern | Effect |
| --- | --- |

Kaolinite*All spectra starting with "Kaolinite"*alunite*Any spectrum containing "alunite" (case-insensitive)Kaolinite*,Alunite*,Calcite*Multiple mineral groups1-10First 10 spectra by index (1-based)1,5,42Specific spectra by index(empty)Use all spectra in the library

## 7.4 Similarity Measures

| Code | Measure | Notes |
| --- | --- | --- |

SAMSpectral Angle MapperRecommended. Insensitive to illumination. Lower angle = better match.BCBray-Curtis DistanceQuantifies compositional dissimilarity. Range [0, 1].SIDSpectral Information DivergenceInformation-theoretic. Sensitive to spectral shape.EDEuclidean DistanceSensitive to both shape and magnitude.IDIntensity DifferenceSum of absolute differences.

All measures use minimum value = best match for the classification image.

## 7.5 Outputs

- Rule Image: Multi-band Float32 GeoTIFF with one band per reference spectrum. Each band contains the similarity (distance) value for every pixel. Lower values indicate better matches. Nodata pixels produce NaN.

- Classification Image (optional): Single-band UInt16 GeoTIFF. Pixel value = 1-based index of the best-matching reference spectrum. Value 0 = unclassified (all-NaN pixel). Supports libraries with up to 65535 spectra.

## 7.6 Automatic Processing

The SAM algorithm automatically handles:

- Wavelength resampling: Reference spectra are interpolated to match image wavelengths when band counts differ.

- Unit conversion: Micrometres and nanometres are auto-detected and converted.

- ENVI no-data handling: Values of −1.23×10³⁴ are replaced with NaN and excluded from comparisons.

- Vectorised computation: All spectral distance measures are computed using NumPy matrix operations rather than per-pixel loops, giving 10–50× speedup over loop-based approaches on typical scene sizes.

# 8. Common Wavelength Ranges

| Application | Start (nm) | End (nm) | Target Features |
| --- | --- | --- | --- |

Clay minerals (AlOH)21002250Kaolinite ~2165, Montmorillonite ~2210Broad SWIR survey20002500General mineral mappingCarbonate minerals23002400Calcite ~2340, Dolomite ~2320Iron oxides8501050Hematite ~860, Goethite ~920Vegetation red edge680750Chlorophyll absorptionVegetation water9001100Leaf water content ~970 nm

# 9. Version History

| Version | Key Changes |
| --- | --- |

2.19Pyrophyllite identification from GMEX pyrophyllite (AusSpec International 2005). New FE17-2066D (doublet ~2066/2078 nm, 2050–2100 nm range, 2060–2090 nm stretch, default 2 features). New FE18-1396W (single sharp OH ~1396 nm, satellite data). New DT Pyrophyllite: classifies via ~2166 nm gate + doublet confirmation, with routing of non-pyrophyllite Al-OH to possible_illite and possible_kandite. FE numbers zero-padded to FE01–FE18 for correct alphabetical ordering in QGIS Processing Toolbox. FE00 Run All updated to FE01–FE18.2.18Kandite group classification from GMEX kandite group comparison spectra (AusSpec International 2005). New FE16-1900W (halloysite water band 1850–1960 nm, broad fitting, depth scale 0–0.35). New DT Kandite: full five-member kandite group classifier (kaolinite, dickite, halloysite hydrated, halloysite dehydrated, nacrite) using FE12-2206W + WoM 2138–2196 nm + FE16-1900W. FE00 Run All updated to FE01–FE16.2.17Dickite identification from GMEX dickite (AusSpec International 2005). New FE14-2178D (dickite Al-OH shoulder 2155–2196 nm), FE15-1384D (dickite OH doublet 1370–1435 nm). New DT Dick-Kaol: dickite vs kaolinite discrimination via Al-OH shoulder position (~2162 nm kaolinite vs ~2178 nm dickite).2.16Kaolinite identification from GMEX kaolinite2 (AusSpec International 2005). New FE12-2206W (primary Al-OH 2162–2245 nm), FE13-1400D (OH doublet 1385–1430 nm). New DT Kaol-Cryst: kaolinite crystallinity classification using Crystallinity Index (depth 2162 / depth 2206). FE0 Run All updated to FE1–FE15.2.10–2.15Toolbar HypPy dropdown menu listing all algorithms grouped as in Processing Toolbox. Qt sender() fix (lambda captures). Spectrum Viewer resampling and x-axis alignment fixes. Various bug fixes and UI improvements.2.9Log Residuals added to Spectral Processing Tools. SAM algorithm vectorised (10–50× speedup); classification output changed to UInt16 (supports >255 spectra); ENVI spectra names parser fixed for USGS splib07 format. Polynomial fit order option (2nd/4th order) added to Step 2b Generate WoM-B and Step 2b Wavelength of Minimum. Step 2b Generate WoM-B correctly labelled (was erroneously showing “Step 2a”).2.8Feature Extraction group (6 - FE) added with 11 one-click tools (FE1–FE11) covering absorption features from 1440 to 2435 nm. Each tool runs WoM, generates a wavelength map with preset colour stretch, and displays a named legend PNG in a floating dock.2.4Toolbox reorganised into current structure. Fixed other-9 mineral class unreachable in dt_2100_2400. Updated citation to van Ruitenbeek et al. (2025). Getting Started now opens USER_GUIDE in browser.2.3Legend dock window made fully resizable. Image scales to fill dock.2.2Legend dock creation moved to postProcessAlgorithm() to fix intermittent display failure.2.0Legend displayed automatically in a floating QDockWidget after processing.1.9Legend output has a dedicated user-supplied filename parameter.1.8Plugin renamed to Hyppy_for_QGIS. Toolbox groups renamed to Step 1 / Step 2.1.5Wavelength Mapping generates a 2D colour legend PNG.1.4New Workflow A with WoM raster generation. New dt_mineral_map classifier (17 classes).1.3Decision tree classifiers: dt_2100_2400, dt_albedo, dt_fedrop, dt_illcryst, dt_ill_kaol.1.2Fixed wavelength reading from ENVI-format rasters in all six spectral processing tools.1.1Six new Spectral Processing Tools: Convex Hull Removal, Band Ratio, Band Ratios, Band Depths, Band Math, Spectra Math.0.9Spectra filter with wildcard, index, and range support. Library browser.0.8Robust ENVI binary reader. Tested against USGS Spectral Library V7 (402 spectra).0.5Auto-converts µm ↔ nm wavelength units.0.4Auto-resamples reference spectra to match image wavelengths.

# 10. References

Crosta, A.P. and McMoore, J., 1989. Enhancement of Landsat Thematic Mapper imagery for residual soil mapping in SW Minas Gerais State, Brazil. Proceedings of the 7th ERIM Thematic Conference on Remote Sensing for Exploration Geology, pp.1173–1187.

Kruse, F.A. et al., 1993. The Spectral Image Processing System (SIPS) — Interactive Visualization and Analysis of Imaging Spectrometer Data. Remote Sensing of Environment, 44(2–3), pp.145–163.

Kokaly, R.F. et al., 2017. USGS Spectral Library Version 7. U.S. Geological Survey Data Series 1035, 61 p.

Bakker, W.H., 2010–2022. HyPy — Hyperspectral Python. University of Twente, Faculty ITC.

AusSpec International, 2005. GMEX: Spectral Geology Reference Manual. AusSpec International Ltd. Spectral library reference data for kaolinite, dickite, halloysite, nacrite, pyrophyllite, and related minerals used in the Feature Extraction (FE12–FE18) and Al-OH mineral decision tree algorithms.

van Ruitenbeek, F.J.A. et al., 2025. A Knowledge-Based Strategy for Interpretation of SWIR Hyperspectral Images of Rocks. Remote Sensing, 17(15), 2555. https://doi.org/10.3390/rs17152555