HypPy for QGIS

Complete Workflow Guide

Version 2.31  |  March 2026

Hyperspectral Analysis for QGIS

# 1.  Overview

HypPy for QGIS provides a comprehensive suite of hyperspectral image analysis tools accessed through the QGIS Processing Toolbox. Algorithms are derived from the HyPy 3.0.1 library by Wim Bakker (University of Twente) and have been extended with mineral-specific feature extraction, decision-tree classification, and an interactive Spectrum Viewer.

## 1.1  Processing Toolbox Organisation

| Group | Contents |
| --- | --- |
| 1 – Workflow Guide | Open this guide in your browser |
| 2 – Generate WoM Rasters | Wavelength of Minimum rasters and wavelength maps |
| 3 – Decision Tree Classification | Rule-based mineral and physical-property classifiers |
| 4 – SAM Classification | Spectral Angle Mapper against ENVI or CSV spectral libraries |
| 5 – Spectral Processing Tools | Band ratios, depths, convex hull removal, band math, log residuals, subset |
| 6 – Feature Extraction | One-click WoM + wavelength map + legend for 19 absorption features |
| 7 – Convert Tools | Convert image, EOS HDF, ASTER HDF, and EMIT L2A files to ENVI format |

## 1.2  Toolbar

A HypPy toolbar is added to the QGIS main window containing a single HypPy dropdown menu button. Clicking it opens a hierarchical menu with seven submenus:

- Workflow Guide — open this guide; workflow summary log

- Generate WoM Rasters — WoM-A/B/C presets, custom WoM, wavelength mapping

- Decision Tree Classifiers — Mineral Map and ten individual DT classifiers

- SAM Classifier — Spectral Angle Mapper

- Spectral Processing Tools — Spectrum Viewer toggle, band math, log residuals, subset, and more

- Feature Extraction Tools — FE00 Run All plus FE01–FE18 and FE-REE individual tools

- Convert Tools — convert Image, EOS HDF, ASTER HDF, or EMIT L2A files to ENVI format

# 2.  Installation

Copy the HypPy_for_QGIS folder into your QGIS user plugins directory:

| Platform | Plugins directory |
| --- | --- |
| Windows | %APPDATA%\QGIS\QGIS3\profiles\<profile>\python\plugins\ |
| Linux | ~/.local/share/QGIS/QGIS3/profiles/<profile>/python/plugins/ |
| macOS | ~/Library/Application Support/QGIS/QGIS3/profiles/<profile>/python/plugins/ |

Enable: Plugins → Manage and Install Plugins → Installed → tick HypPy for QGIS → OK

Reload after update: Plugins → Manage and Install Plugins → HypPy for QGIS → Uninstall/Reinstall, or use the Plugin Reloader plugin.

Note: GDAL and NumPy are required. Both are included in the standard QGIS installation. No additional Python packages are needed.

# 3.  Spectrum Viewer

The Spectrum Viewer is an interactive dock panel modelled on the HypPy 3.0.1 SpecLib Viewer. It allows you to compare pixel spectra extracted from a hyperspectral raster against reference spectra from an ENVI or ASCII spectral library.

## 3.1  Opening the Viewer

- Open the HypPy menu, go to Spectral Processing Tools, and click Spectrum Viewer. The item is checkable — a tick mark shows when the dock is visible.

- The dock can be docked to any edge of the QGIS window or floated as a standalone panel.

## 3.2  Picking a Pixel Spectrum

- Select a hyperspectral raster layer in the Layers panel.

- ▶ Pick pixel — click the button to activate the map tool.

- Click on any pixel in the map canvas. The spectrum is extracted and plotted immediately.

- Right-click on the map, or press Esc, to deactivate the tool.

- Clear pixel — removes the pixel spectrum from the plot.

Wavelength metadata: Wavelengths are read from the ENVI .hdr file associated with the raster. If no wavelength metadata is found, band numbers are used on the X axis.

## 3.3  Loading a Spectral Library

Select a library type using the radio buttons, then click Browse…:

| Library type | File format |
| --- | --- |
| ENVI Speclib (.sli / .hdr) | Binary ENVI spectral library. Wavelength units are read from the .hdr file. |
| ASCII file (.csv / .txt) | Single CSV or text file with wavelength column and one or more spectrum columns. |
| ASCII directory | All .csv and .txt files in the selected directory are loaded as individual spectra. |
| ASCII directory (recursive) | As above, but searches all subdirectories. |

After loading, spectra names appear in the list. Select one or more names (Ctrl+click for multiple, Shift+click for range) and click Plot.

## 3.4  Menubar

The Spectrum Viewer has a full menubar matching the HypPy SpecLib Viewer:

### File menu

| Item | Action |
| --- | --- |
| Open library… | Open a spectral library (respects the selected Library type) |
| Close viewer | Hide the Spectrum Viewer dock panel |

### Select menu

| Item | Action |
| --- | --- |
| Select all | Select all spectra in the library list |
| Deselect all | Clear the selection |

### View menu

| Item | Action |
| --- | --- |
| No hull removal | Plot spectra without continuum removal (default) |
| CR by division | Plot continuum-removed spectra: spectrum ÷ convex hull |
| CR by subtraction | Plot continuum-removed spectra: 1 + (spectrum − convex hull) |
| Plot spectra | Plot selected spectra (same as the Plot button) |
| Plot + average | Plot selected spectra and add their average as an additional curve |
| View values | Switch to the Values tab showing the wavelength/value table for the first selected spectrum |
| Legend on / off | Show or hide the draggable legend box on the plot |
| Set xlabel | Override the X axis label (default: wavelength (Micrometers)) |
| Set ylabel | Override the Y axis label (default: reflectance (normalised 0–1)) |
| Set title | Add a title above the plot |
| Set axes | Manually set X and Y axis limits |
| Reset axes | Return to automatic axis scaling |
| Reset legend position | Return the legend to its default top-right position |
| REE Nd markers (580/740/800/870 nm) | Toggle vertical marker lines at the four neodymium absorption bands |

### Math menu

Run expression on S… — evaluate a Python/NumPy expression on the selected spectrum. The spectrum object S is available with the following methods:

| Expression | Result |
| --- | --- |
| S.nohull() | Continuum-removed spectrum (division) |
| S[2200.0] | Value at wavelength 2200.0 nm (nearest band) |
| S.mean() | Mean reflectance over all bands |
| S.smooth(5) | Savitzky-Golay smoothed spectrum (window 5) |
| S.gradient() | First derivative of the spectrum |
| S1 + S2 | Band-wise sum of two Spectrum objects (auto-resampled) |
| S / 2.0 | Scale spectrum by scalar |

## 3.5  The Plot

All spectra (pixel and library) are plotted on a single shared Y axis normalised to 0–1. Each series is independently scaled to its own minimum and maximum, allowing direct comparison of spectral shapes regardless of absolute reflectance level or instrument units.

| Feature | Description |
| --- | --- |
| Shared Y axis (0–1) | Pixel spectrum (DN or reflectance) and library spectra (fractional reflectance) are all independently normalised to 0–1 for direct visual comparison |
| Crosshair | Hover the mouse over the plot area to see a crosshair and wavelength/value tooltip |
| Draggable legend | Click and drag the legend box to any position on the plot. Right-click → Reset legend position to restore the default top-right position |
| REE Nd markers | Vertical red lines at 580, 740, 800 and 870 nm mark the neodymium absorption bands (Boesche et al. 2015) |
| Convex hull removal | Remove hull checkbox or View menu radio buttons apply continuum removal to library spectra before plotting |
| Right-click menu | Copy pixel spectrum as CSV, clear library spectra, clear all, reset legend position |

## 3.6  Values Tab

The Values tab shows the wavelength and reflectance value table for the currently selected spectrum. Click Copy as CSV to copy the table to the clipboard.

# 4.  Generate WoM Rasters  (Group 2)

Wavelength of Minimum (WoM) rasters record the wavelength position and depth of absorption features on a per-pixel basis. They are the core input for wavelength mapping and feature extraction.

## 4.1  Step 2a — Generate All WoM Rasters

Runs WoM-A (750–1300 nm), WoM-B (1850–2100 nm), and WoM-C (2100–2400 nm) in a single step. Recommended for new datasets.

## 4.2  Step 2b — Wavelength of Minimum (Custom Range)

Runs WoM for a user-specified wavelength range. Useful for individual feature investigation.

| Parameter | Description |
| --- | --- |
| Start / End wavelength (nm) | Wavelength range to search for absorption minimum |
| Continuum removal mode | Division (default) or subtraction continuum removal before minimum detection |
| Number of features | Number of absorption minima to extract per pixel (1–9) |
| Broad feature mode | Use parabola fitting to locate broad features; narrow features use direct minimum |
| Use BBL | Respect Bad Band List in ENVI header — masked bands are excluded from minimum search |
| Polynomial fit order | 2nd order (quadratic) or 4th order (quartic) — affects WoM-B only |

## 4.3  Step 2c — Wavelength Mapping

Converts a WoM raster (band 1 = wavelength, band 2 = depth) into a colour-coded RGB image using a rainbow palette. The colour stretch is mapped between user-supplied minimum and maximum wavelengths.

# 5.  Feature Extraction  (Group 6)

Feature Extraction tools are one-click pipelines that run WoM, generate a wavelength map with a preset colour stretch, save a named legend PNG, and display the legend as a floating dock panel.

FE0 — Run All: FE0 runs FE1 through FE11 in a single step, generating all standard SWIR mineral feature maps from one dialog.

## 5.1  Standard SWIR Features (FE1–FE11)

| Tool | WoM range (nm) | Map stretch (nm) | Target feature |
| --- | --- | --- | --- |
| FE1 – 1480W | 1440–1520 | 1471–1491 | Carbonate / Chlorite OH |
| FE2 – 1550W | 1510–1610 | 1520–1563 | Carbonate CO₃ |
| FE3 – 1760W | 1730–1790 | 1751–1764 | Carbonate CO₃ |
| FE4 – 2080D | 2060–2100 | 2075–2085 | Carbonate doublet |
| FE5 – 2160D | 2138–2179 | 2159–2166 | Al-OH / Pyrophyllite doublet |
| FE6 – 2200W | 2120–2245 | 2185–2215 | Al-OH / Kaolinite / Illite |
| FE7 – 2250W | 2230–2280 | 2248–2268 | Fe-OH / Chlorite |
| FE8 – 2290W | 2270–2320 | 2279–2338 | Mg-OH / Chlorite |
| FE9 – 2320W | 2295–2345 | 2300–2340 | Mg-OH / Chlorite / Antigorite |
| FE10 – 2350W | 2310–2370 | 2320–2366 | Mg-OH / Amphibole |
| FE11 – 2390W | 2375–2435 | 2377–2406 | Mg-OH / Talc / Tremolite |

## 5.2  Extended Clay/Kandite Features (FE12–FE18)

| Tool | WoM range (nm) | Map stretch (nm) | Target feature |
| --- | --- | --- | --- |
| FE12 – 2206W | 2162–2245 | 2197–2215 | Kaolinite Al-OH (fine position) |
| FE13 – 1400D | 1385–1430 | 1395–1420 | Kaolinite OH stretch doublet (1400/1412 nm) |
| FE14 – 2178D | 2155–2196 | 2168–2190 | Dickite Al-OH shoulder |
| FE15 – 1384D | 1370–1435 | 1378–1425 | Dickite OH doublet (1384 nm position) |
| FE16 – 1900W | 1850–1960 | 1870–1940 | Halloysite interlayer water band |
| FE17 – 2066D | 2050–2100 | 2060–2090 | Pyrophyllite Al-OH doublet (2066 nm) |
| FE18 – 1396W | 1380–1420 | 1388–1406 | Pyrophyllite OH (1396 nm short-wave position) |

Note — atmospheric water bands: FE13, FE15, FE16, and FE18 target wavelength regions near atmospheric water vapour absorptions. In airborne data these regions are typically masked by BBL. In satellite data with good atmospheric correction (EnMAP, PRISMA) the features are often resolvable.

## 5.3  REE Neodymium VNIR Feature (FE-REE)

The FE-REE tool targets four characteristic VNIR absorption bands of the Nd³⁺ ion as documented by Boesche et al. (2015). These features are diagnostic for neodymium-bearing minerals in carbonatite and alkaline intrusive terranes.

| Band (nm) | Ion transition | Map colour | Mineral context |
| --- | --- | --- | --- |
| ~580 | Nd³⁺ ⁴G₅/₂ + ⁴G₇/₂ | Blue/violet | Monazite, allanite, apatite |
| ~740 | Nd³⁺ ⁴F₇/₂ + ⁴S₃/₂ | Cyan | Monazite, bastnäsite |
| ~800 | Nd³⁺ ⁴F₅/₂ + ²H₉/₂ | Green | Monazite, xenotime (weak) |
| ~870 | Nd³⁺ ⁴F₃/₂ | Orange/red | Monazite, carbonatite matrix |

| Parameter | Value |
| --- | --- |
| WoM search range | 530–910 nm (covers all four bands) |
| Colour stretch | 560–900 nm |
| Default features | 4 (extract all four Nd absorption bands) |
| Typical depth | <5 % (shallow — use low depth threshold) |

Interpretation note: The Nd absorptions are shallow (<5%) compared to SWIR clay/carbonate features — good atmospheric correction and a log-residuals pre-processing step are recommended. The ~580 nm band can be masked by Fe³⁺ absorption (goethite, hematite); confirm against the 740 and 870 nm bands. All four bands present simultaneously strongly indicates Nd-bearing phases.

# 6.  Decision Tree Classification  (Group 3)

Decision tree algorithms classify pixels using rule-based thresholds applied to WoM raster outputs. All algorithms require previously generated WoM-B and/or WoM-C rasters as input (not the raw hyperspectral cube).

| Algorithm | Inputs | Output classes |
| --- | --- | --- |
| DT Mineral Map | WoM-B + WoM-C | 17 mineral classes (illite, kaolinite, chlorite, calcite, dolomite, and more) |
| DT 2100–2400 | WoM-C | 9 mineral classes for the 2100–2400 nm range |
| DT Albedo | Raw cube | 5 brightness classes (very dark → very bright) using per-pixel mean reflectance and dynamic quartile thresholds |
| DT Fedrop | Raw cube | 5 iron-drop intensity classes using R(1600)/R(1310) band ratio |
| DT Illcryst | WoM-B | 5 illite crystallinity classes based on WoM-B wavelength position |
| DT Ill-Kaol | WoM-B + WoM-C | 3 classes: illite-dominant / mixed / kaolinite-dominant |
| DT Dick-Kaol | WoM-B + WoM-C | 3 classes: dickite-dominant / mixed / kaolinite-dominant |
| DT Kandite | WoM-B | 4 kandite mineral classes |
| DT Pyrophyllite | WoM-B | 3 classes: pyrophyllite / alunite / other Al-OH |

# 7.  SAM Classification  (Group 4)

The Spectral Angle Mapper classifies pixels by measuring the spectral angle between each pixel and a set of reference spectra. SAM is insensitive to illumination variations, making it robust for mineral identification.

## 7.1  Inputs

- Hyperspectral cube — with wavelength metadata in ENVI header format

- Spectral library — ENVI binary (.sli + .hdr) or CSV text (.csv / .txt)

## 7.2  Spectra Filter

| Filter pattern | Effect |
| --- | --- |
| Kaolinite* | All spectra starting with "Kaolinite" |
| *alunite* | Any spectrum containing "alunite" (case-insensitive) |
| Kaolinite*,Alunite* | Multiple comma-separated patterns |
| 1-10 | First 10 spectra by index (1-based) |
| 1,5,42 | Specific spectra by index number |
| (empty) | Use all spectra in the library |

## 7.3  Similarity Measures

| Code | Measure | Notes |
| --- | --- | --- |
| SAM | Spectral Angle Mapper | Recommended. Insensitive to illumination. |
| BC | Bray-Curtis Distance | Quantifies compositional dissimilarity. Range [0,1]. |
| SID | Spectral Information Divergence | Information-theoretic. Sensitive to spectral shape. |
| ED | Euclidean Distance | Sensitive to shape and magnitude. |
| ID | Intensity Difference | Sum of absolute differences. |

# 8.  Spectral Processing Tools  (Group 5)

| Tool | Description |
| --- | --- |
| Convex Hull Removal | Per-pixel continuum removal using convex hull fitting. Division or subtraction mode. |
| Band Ratio | Single ratio of two bands by wavelength: R(λ1) / R(λ2). |
| Band Ratios | Sequential ratio of all bands with optional delta: outputs an (n−1) band image. |
| Band Depths | Per-band absorption feature depths relative to a continuum. |
| Band Math | Evaluate a NumPy expression over image bands using variables i1, i2 … or i1(λ) for nearest-wavelength lookup. |
| Spectra Math | Evaluate a per-pixel expression using Spectrum objects S1, S2 … with full arithmetic and method support. |
| Log Residuals | 3-pass Log Residuals or Kwik Residuals normalisation. Optional albedo and RLUB outputs. BIP-interleave streaming for large files. |
| Subset | Spatial and/or spectral subset of an ENVI image. Spatial bounds can be entered as pixel sample/line numbers (0-based) or as a map coordinate extent rectangle (converted to pixels via the geotransform; works for north-up and rotated images). Spectral selection by wavelength range in nm or HyPy-syntax band index list. Optional BBL exclusion and wavelength sorting. Output in BSQ/BIL/BIP with optional data type conversion. Wavelengths, FWHM, BBL, and map info are preserved and updated in the output ENVI .hdr sidecar. |

## 8.1  Subset

Extracts a spatial and/or spectral subset from an ENVI hyperspectral image. The output is a new ENVI flat-binary file with a complete sidecar .hdr containing updated wavelengths, FWHM, BBL, band names, and corrected map georeferencing.

### 8.1.1  Spatial Subset

Two modes are available, selected by the Use map coordinates checkbox:

Pixel mode (“Use map coordinates” unticked, default) — enter First/Last Sample (column, 0-based) and First/Last Line (row, 0-based). Set Last Sample or Last Line to 0 to use the full image extent in that dimension.

Map coordinate mode (“Use map coordinates” ticked) — draw or enter a bounding box in the Spatial Extent field. The box is converted to pixel sample/line indices using the GDAL geotransform (works for north-up and rotated images). The log reports the map extent and derived pixel bounds for verification.

### 8.1.2  Spectral Subset

Spectral selection uses one of two fields (Band Index List takes priority; leave both blank to keep all bands):

Wavelength Range — enter min-max in nm, e.g. 1000-2400. Only bands whose centre wavelength falls within the range are included.

Band Index List — HyPy-syntax 0-based indices, e.g. 4 (band 4 only), 6-8 (bands 6–8), <3 (first 4 bands), >100 (band 100 to last), 1 4 6-8 (bands 1, 4, 6–8).

### 8.1.3  Options

Sort bands by ascending wavelength — reorders bands by wavelength before subsetting (default on).

Exclude bad bands (BBL) — removes bands flagged as bad in the ENVI header Bad Band List (default on).

Output Interleave Format — BSQ (default), BIL, or BIP.

Output Data Type — keep (same as input) or convert to uint8, int16, uint16, int32, uint32, float32, or float64.

# 9.  Convert Tools  (Group 7)

The Convert Tools group provides four algorithms that read proprietary or complex satellite formats and write ENVI flat-binary output alongside the source file. All converters use GDAL and the osgeo Python bindings; no additional Python packages are required beyond the standard QGIS installation.

| Tool | Description |
| --- | --- |
| Convert Image to ENVI | Converts a TIFF, JPEG, PNG, or BMP image to ENVI flat-binary format using GDAL. Georeferencing embedded in GeoTIFF tags is preserved in the output ENVI header. |
| Convert EOS HDF to ENVI | Extracts every EOS_SWATH subdataset from an EOS HDF4 file and writes a separate ENVI binary + .hdr pair per band, named <basename>_<BandName>_<SetName>. Requires GDAL built with HDF4 support. |
| Convert ASTER HDF to ENVI | Extracts ImageData, Band, and Temperature subdatasets from an ASTER HDF file. Swath geometry is re-projected using gdalwarp; grid geometry uses gdal_translate. Output files are written alongside the source HDF, named <base>_<BandDescription>. |
| Convert EMIT L2A to ENVI | Reads an EMIT L2A NetCDF4 reflectance file and writes four ENVI outputs alongside it: _refl (reflectance cube, BIL float32, with wavelengths/fwhm/bbl), _lonlat (longitude + latitude, BSQ 2-band), _elev (elevation, BSQ 1-band), and _glt (GLT x/y for orthorectification, BSQ int32 2-band). Requires the netCDF4 Python package (pip install netCDF4). |

# 10.  Common Wavelength Ranges

| Application | Start (nm) | End (nm) | Target features |
| --- | --- | --- | --- |
| REE — Neodymium VNIR | 530 | 910 | Nd³⁺ absorptions at ~580, 740, 800, 870 nm (Boesche et al. 2015) |
| Iron oxides VNIR | 850 | 1050 | Hematite ~860 nm, Goethite ~920 nm |
| Vegetation red edge | 680 | 750 | Chlorophyll absorption edge |
| Vegetation water | 900 | 1100 | Leaf water content ~970 nm |
| Kaolinite OH doublets | 1370 | 1435 | Kaolinite 1400/1412 nm, Dickite 1384 nm, Pyrophyllite 1396 nm |
| Halloysite water | 1850 | 1960 | Interlayer water ~1900 nm |
| Carbonate / chlorite | 2060 | 2110 | 2080 nm carbonate doublet |
| Al-OH minerals | 2120 | 2250 | Kaolinite ~2165 nm, Illite ~2200 nm, Montmorillonite ~2210 nm |
| Fe-OH / Mg-OH minerals | 2230 | 2440 | Chlorite 2250–2290 nm, Amphibole ~2350 nm, Talc ~2390 nm |
| Carbonate minerals | 2300 | 2400 | Calcite ~2340 nm, Dolomite ~2320 nm |

# 11.  Version History

| Ver. | Title | Key changes |
| --- | --- | --- |
| 2.31 | Subset: fix NameError | Fixed NameError: name _update_map_info is not defined. The function def line was accidentally dropped when _map_extent_to_pixels was inserted in v2.30, leaving the function body present but unreachable. |
| 2.30 | Subset: map coordinate mode | New “Use map coordinates” checkbox. When ticked, spatial subset is defined by a map-coordinate extent rectangle (Spatial Extent field) instead of pixel sample/line numbers. The bounding box is converted to pixel indices via the GDAL geotransform; works for north-up and rotated images. Bounds are clamped to the image and logged for verification. |
| 2.29 | Subset: correct output coordinates | The ENVI .hdr map info was written with the original image tie-point, so subsets were georeferenced to the wrong location. New _update_map_info() recalculates the easting/northing for the new upper-left pixel using the geotransform (with half-pixel centre offset). The GDAL geotransform on the output dataset was already correct; the .hdr map info now matches it. |
| 2.28 | Subset: fix numpy array truth-value errors | Fixed ValueError “The truth value of an array with more than one element is ambiguous” that occurred whenever wavelength metadata was present (returned as a numpy array). Bare truthiness checks replaced with explicit is not None and len() guards in the sort_wav branch, reindex() helper, and sel_wavelengths / sel_fwhm / sel_bbl / sel_band_names ternaries. |
| 2.27 | Spectrum Viewer moved into menu | The standalone Spectrum Viewer toolbar button has been removed. Spectrum Viewer is now the first entry in the Spectral Processing Tools submenu, separated from the processing algorithms by a divider. The item is checkable and stays in sync with the dock’s show/hide state. The toolbar now contains only the single HypPy dropdown button. |
| 2.26 | Hierarchical toolbar dropdown menu | HypPy toolbar dropdown converted from a flat list with bold disabled headers to a hierarchical menu with seven submenus: Workflow Guide, Generate WoM Rasters, Decision Tree Classifiers, SAM Classifier, Spectral Processing Tools, Feature Extraction Tools, and Convert Tools. The _MENU_STRUCTURE data structure makes adding new algorithms or sections a single-line edit. |
| 2.25 | Subset tool | New Subset algorithm (E08) in Spectral Processing Tools. Supports spatial clipping (sample/line start–end), spectral selection by wavelength range or HyPy-syntax band list, wavelength sorting, BBL exclusion, and output in BSQ/BIL/BIP with optional data type conversion. Full ENVI header sidecar written including wavelengths, fwhm, bbl, band names, and map info. |
| 2.24 | Convert Tools group (Group 7) | New Convert Tools group (Group 7) with four algorithms: Convert Image to ENVI (TIFF/JPEG/PNG/BMP via GDAL), Convert EOS HDF to ENVI (all EOS_SWATH subdatasets), Convert ASTER HDF to ENVI (ImageData/Band/Temperature subdatasets with gdalwarp/gdal_translate), and Convert EMIT L2A to ENVI (reflectance, lon/lat, elevation, and GLT outputs from NetCDF4). Windows GDAL subprocess issues resolved using osgeo_utils.gdal_pansharpen pattern. |
| 2.23 | Spectrum Viewer stability fixes | Fixed RuntimeError when picking pixels or stopping the pick tool after the dock widget has been closed or the plugin reloaded — sip.isdeleted() guards added to _read_pixel, _on_tool_activated, and _on_tool_deactivated. |
| 2.21 | Negative reflectance masking in Spectrum Viewer | Pixel spectrum values below zero (noisy detector bands, typically beyond ~2400 nm) are now masked to NaN and displayed as gaps in the plot, preventing the Y axis from being pulled below zero and obscuring absorption features. |
| 2.20 | Spectrum Viewer View menu + REE feature extraction | Spectrum Viewer: full File / Select / View / Math menubar matching HypPy SpecLib Viewer. View menu adds CR modes, Plot + average, View values, Legend on/off, Set xlabel / ylabel / title / Set axes / Reset axes, Reset legend position, REE Nd marker toggle. Draggable legend: click-and-drag legend box to any position; right-click → Reset legend position. Y axis normalised to 0–1 per-series for direct shape comparison; right axis removed; single shared axis. FE-REE: new Neodymium VNIR feature extraction (530–910 nm WoM, 560–900 nm stretch, 4 Nd absorption bands per Boesche et al. 2015). Toolbar dropdown updated with FE12–FE18 mineral descriptions and FE-REE entry. |
| 2.19 | HypPy SpecLib Viewer aesthetics + library type selector | Spectrum Viewer restyled to match HypPy 3.0.1 matplotlib figure window: matplotlib tab10 colour cycle, white background, grey grid, black spines, inward ticks, line-handle legend (8pt). Wavelength units propagated from ENVI header to X axis label and Values table. Library type selector (ENVI Speclib / ASCII file / ASCII directory / recursive). Math expression dock. Draggable legend (v2.20 finalised). |
| 2.18 | Spectrum Viewer toolbar integration | HypPy toolbar extended with full dropdown menu listing all algorithms grouped as in Processing Toolbox. Fixed Qt sender() AttributeError using lambda captures. Fixed Spectrum Viewer library spectra resampling and X-axis alignment bug. |
| 2.17 | FE12–FE18 kandite and pyrophyllite features | Added FE12 (2206W Kaolinite), FE13 (1400D Kaolinite doublet), FE14 (2178D Dickite), FE15 (1384D Dickite doublet), FE16 (1900W Halloysite), FE17 (2066D Pyrophyllite), FE18 (1396W Pyrophyllite). All include atmospheric water band notes where relevant. |
| 2.12–2.16 | Spectrum Viewer initial versions | Interactive Spectrum Viewer dock: pixel pick map tool, ENVI library loading, spectrum plotting, convex hull removal, math expression evaluation, Values tab. |
| 2.11 | Full HyPy Spectrum class + Image.__call__ | Spectra Math: float wavelength indices S1[2200.0] and S1(2200.0); __coerce__ auto-resamples; full numpy method set; spectral shape methods (hull, nohull, peaks, localmin, minwav, smooth, gradient). Band Math: i1(2200.0) wavelength lookup. Spectrum shared in core/spectrum.py. |
| 2.10 | Wavelength unit normalisation | All algorithms auto-convert wavelengths to nm. Handles nm (no change), µm/Microns (×1000), Wavenumber/cm⁻¹ (10⁷/value). Heuristic: values all < 100 assumed µm. |
| 2.9 | Log Residuals + SAM improvements | New Log Residuals tool (3-pass + Kwik Residuals, BIP streaming). SAM vectorised (10–50× speedup), UInt16 output, nodata handling. Polynomial fit order (2nd/4th) for WoM-B. |
| 2.8 | Feature Extraction group (FE1–FE11) | 6 – Feature Extraction with 11 one-click tools covering 1440–2435 nm. Legend PNG auto-displayed in floating dock. Vectorised colour mapping. |
| 2.4 | Toolbox restructure | Groups renamed: Step 1/2/3/4/5/6. Fixed dt_2100_2400 other-9 class. Updated citations. |
| 2.3 | Resizable legend dock | Legend dock fully resizable by dragging edges. Legend dock creation moved to postProcessAlgorithm(). Plugin renamed Hyppy_for_QGIS. |
| 2.0 | Legend floating dock | Legend displayed in floating QDockWidget after processing. |
| 1.4 | Workflow A + DT Mineral Map | WoM-A/B/C generation tools. dt_mineral_map 17-class classifier. |
| 1.1 | Spectral Processing Tools | Convex Hull Removal, Band Ratio, Band Ratios, Band Depths, Band Math, Spectra Math. |

# 11.  References

Bakker, W.H., 2010–2022. HyPy — Hyperspectral Python. University of Twente, Faculty ITC.

Boesche, N.K., Rogass, C., Lubitz, C., Brell, M., Herrmann, S., Mielke, C., Tonn, S., Appelt, O., Altenberger, U. and Kaufmann, H., 2015. Hyperspectral REE (Rare Earth Element) Mapping of Outcrops — Applications for Neodymium Detection. Remote Sensing, 7(5), 5160–5186. https://doi.org/10.3390/rs70505160

Kokaly, R.F. et al., 2017. USGS Spectral Library Version 7. U.S. Geological Survey Data Series 1035, 61 p.

Kruse, F.A. et al., 1993. The Spectral Image Processing System (SIPS) — Interactive Visualization and Analysis of Imaging Spectrometer Data. Remote Sensing of Environment, 44(2–3), pp.145–163.

van Ruitenbeek, F.J.A. et al., 2025. A Knowledge-Based Strategy for Interpretation of SWIR Hyperspectral Images of Rocks. Remote Sensing, 17(15), 2555. https://doi.org/10.3390/rs17152555