QGIS Hyppy Plugin

Hyperspectral Analysis Tools

Complete Workflow Guide

Version 1.2  |  February 2026

# 1. Overview

The QGIS Hyppy plugin provides nine hyperspectral analysis algorithms ported from the HyPy (Hyperspectral Python) toolkit by Wim Bakker at the University of Twente. The tools are accessed via the QGIS Processing Toolbox.

This guide covers the complete workflow for all nine tools, including installation, configuration, and interpretation of results.

## Plugin Structure

| Algorithm | Group | Purpose |
| --- | --- | --- |
| Spectral Angle Mapper (SAM) | Classification | Classify pixels against a spectral library |
| Wavelength of Minimum v2 | Step 1 - Min Wavelength | Extract absorption feature wavelengths and depths |
| Wavelength Mapping | Step 2 - Wav Mapping | Create RGB visualisation from absorption features |
| Convex Hull Removal | Spectral Processing | Continuum removal using convex hull fitting |
| Band Ratio | Spectral Processing | Single-band ratio (Band1/Band2) by wavelength |
| Band Ratios (Sequential) | Spectral Processing | All-band sequential ratios with delta |
| Band Depths | Spectral Processing | Per-band absorption feature depths |
| Band Math | Spectral Processing | NumPy expression over image bands (i1, i2...) |
| Spectra Math | Spectral Processing | Per-pixel spectral expression (S1, S2...) |

# 2. Installation

Copy the QGIS_Hyppy_v_0_9 folder to your QGIS plugins directory:

Windows: %APPDATA%\QGIS\QGIS3\profiles\<profile>\python\plugins\

Linux: ~/.local/share/QGIS/QGIS3/profiles/<profile>/python/plugins/

macOS: ~/Library/Application Support/QGIS/QGIS3/profiles/<profile>/python/plugins/

Restart QGIS, then enable the plugin via Plugins > Manage and Install Plugins. The Hyppy tools will appear in the Processing Toolbox under the Hyppy provider.

## Requirements

- QGIS 3.16 or later

- Python 3.x with NumPy and SciPy (included with QGIS)

- GDAL/OGR (included with QGIS)

# 3. Workflow A: SAM Classification

The Spectral Angle Mapper classifies image pixels by measuring the spectral angle between each pixel and a set of reference spectra. SAM is insensitive to illumination variations, making it robust for mineral identification.

## 3.1  Prepare Input Data

- Hyperspectral image with wavelength metadata in ENVI header format. The image must have wavelength values for each band.

- Spectral library in ENVI binary format (.sli + .hdr) or CSV text format (.csv, .txt).

## 3.2  Browse the Spectral Library (Optional)

To see what spectra are available before running classification:

- Open SAM from Processing Toolbox > Hyppy > Classification > Spectral Angle Mapper

- Select your spectral library file

- Check "List available spectra names (does not run SAM)"

- Click Run. All spectra will be listed with index numbers in the log.

## 3.3  Configure the Spectra Filter

Use the filter to select specific reference spectra. This is essential when using large libraries like the USGS Spectral Library Version 7 (402 minerals).

| Filter Pattern | Effect |
| --- | --- |
| Kaolinite* | All spectra starting with 'Kaolinite' |
| *alunite* | Any spectrum containing 'alunite' (case-insensitive) |
| Kaolinite*,Alunite*,Calcite* | Multiple mineral groups |
| 1-10 | First 10 spectra by index (1-based) |
| 1,5,42 | Specific spectra by index |
| (empty) | Use all spectra in the library |

## 3.4  Select Similarity Measure

| Code | Measure | Notes |
| --- | --- | --- |
| SAM | Spectral Angle Mapper | Recommended. Insensitive to illumination. Measures angle between spectra. |
| BC | Bray-Curtis Distance | Quantifies compositional dissimilarity. Range [0,1]. |
| SID | Spectral Information Divergence | Information-theoretic. Sensitive to spectral shape. |
| ED | Euclidean Distance | Geometric distance. Sensitive to shape and magnitude. |
| ID | Intensity Difference | Sum of absolute differences. |

## 3.5  Outputs

- Rule Image: Multi-band raster with similarity values for each reference spectrum. Lower values indicate better matches.

- Classification Image: Single-band raster with class assignments (0 = unclassified, 1+ = class index).

## 3.6  Automatic Processing

The SAM algorithm automatically handles several common data compatibility issues:

- Wavelength resampling: Reference spectra are interpolated to match image wavelengths when band counts differ.

- Unit conversion: Micrometres and nanometres are auto-detected and converted (e.g. USGS library at 0.35–2.50 µm to image at 418–2445 nm).

- ENVI no-data handling: Values of -1.23e34 are replaced with NaN and excluded from comparisons.

# 4. Workflow B: Mineral Mapping (Wavelength Analysis)

This two-step workflow extracts absorption feature positions from hyperspectral data and creates colour-coded maps for mineral identification.

## 4.1  Step 1: Wavelength of Minimum v2

Analyses each pixel’s spectrum within a defined wavelength range. Applies convex hull continuum removal, locates local minima, and fits parabolas for sub-band-resolution wavelength and depth values.

Parameters:

| Parameter | Typical Value | Notes |
| --- | --- | --- |
| Input Image | Hyperspectral raster | Must have wavelength metadata |
| Start Wavelength (nm) | 2000 | Begin analysis window |
| End Wavelength (nm) | 2500 | End analysis window |
| Continuum Removal | Division | Division recommended for minerals |
| Number of features (1–9) | 1–3 | 1 for single mineral, 3+ for multi-mineral |
| Broad feature fitting | Off | On for broad absorption features |
| Use Bad Band List | On if available | Excludes noisy bands flagged in ENVI metadata |

Output:

A multi-band GeoTIFF containing wavelength/depth pairs for each requested feature. Band 1 = interpolated min. wavelength, Band 2 = interpolated depth, and so on for additional features. Features are sorted by depth (deepest first).

## 4.2  Step 2: Wavelength Mapping

Takes the Step 1 output and creates an RGB image. Colour (hue) represents absorption wavelength position and brightness represents absorption depth.

Parameters:

| Parameter | Typical Value | Notes |
| --- | --- | --- |
| Input (Step 1 output) | Step 1 GeoTIFF | Uses Band 1 and Band 2 directly |
| Min Wavelength for Display (nm) | 2100 | Blue end of colour stretch |
| Max Wavelength for Display (nm) | 2350 | Red end of colour stretch |
| Min Depth for Display | 0.0 | Lower bound of brightness stretch |
| Max Depth for Display | 0.2 | Adjust for brightness (try 0.05–0.3) |

Output:

A 3-band RGB GeoTIFF. Hue encodes wavelength position (blue→green→yellow→red across the wavelength range) and brightness encodes absorption depth.

# 5. Workflow C: Spectral Processing Tools

The six spectral processing tools can be applied independently to any hyperspectral raster. All tools read wavelength metadata from the ENVI domain first, with fallback to the default metadata domain and numeric band descriptions.

## 5.1 Convex Hull Removal (Continuum Removal)

Removes the spectral continuum from each pixel spectrum using a convex hull fit, isolating individual absorption features for analysis.

Parameters:

| Parameter | Description / Values |
| --- | --- |
| Input Image | Hyperspectral raster with wavelength metadata |
| Mode | Divide: spectrum / hull (absorptions < 1.0) ¦ Subtract: 1 + (spectrum - hull) |
| Cutoff Wavelength (nm) | Restrict to bands below this wavelength. Set to 0 to use all bands. |
| Use Bad Band List | Exclude bands flagged in ENVI BBL metadata |

Output: Multi-band GeoTIFF matching the input band count (up to cutoff). Divide mode: values around 1.0, absorption features < 1.0. Subtract mode: similar centring around 1.0.

## 5.2 Band Ratio

Computes a single-band ratio image: Numerator band / Denominator band. Bands are specified by wavelength in nm; the nearest available band is selected automatically.

Parameters:

| Parameter | Description / Values |
| --- | --- |
| Input Image | Hyperspectral raster |
| Numerator Wavelength (nm) | Wavelength of the numerator band. Nearest band selected. |
| Denominator Wavelength (nm) | Wavelength of the denominator band. Nearest band selected. |

Output: Single-band floating-point GeoTIFF. If no wavelength metadata is present, values are treated as 1-based band indices.

## 5.3 Band Ratios (Sequential)

Computes sequential band ratios across the full spectrum: output[i] = input[i] / input[i + delta]. Useful for highlighting spectral gradients and transitions.

Parameters:

| Parameter | Description / Values |
| --- | --- |
| Input Image | Hyperspectral raster |
| Delta | Band offset between numerator and denominator. Default = 1 (each band vs the next). |

Output: Multi-band GeoTIFF with (n_bands - delta) bands. Wavelength metadata is preserved for the numerator bands.

## 5.4 Band Depths

Computes per-band absorption feature depths. For each centre band i: top = (band[i-delta] + band[i+delta]) / 2, depth = (top - band[i]) / top. Positive values indicate absorption.

Parameters:

| Parameter | Description / Values |
| --- | --- |
| Input Image | Hyperspectral raster |
| Delta | Distance to shoulder bands used to compute the continuum top. Default = 1. |

Output: Multi-band GeoTIFF with (n_bands - 2*delta) bands. Wavelengths assigned to centre band positions.

## 5.5 Band Math

Evaluates an arbitrary Python/NumPy expression over one or more input rasters to produce a single-band output. Input images are referenced as i1, i2, ... (up to 20).

Band access methods:

- i1.get_band(n) - returns band n as a 2D array (0-based index)

- i1.band(n) - same but 1-based index

- i1[n] - same as get_band(n)

- i1.bands - number of bands

- i1.wavelength - wavelength array (or None if not available)

NumPy functions are available: sqrt, log, sin, cos, abs, minimum, maximum, etc.

Expression examples:

| Expression | Description |
| --- | --- |
| i1.get_band(10) / i1.get_band(20) | Band ratio (0-based, bands 11 and 21) |
| (i1.get_band(30) - i1.get_band(20)) / (i1.get_band(30) + i1.get_band(20)) | Normalised difference (NDVI-style) |
| i1.get_band(0) - i2.get_band(0) | Band 1 difference between two images |
| log(i1.get_band(4)) | Natural log of band 5 |

Output: Single-band floating-point GeoTIFF. The expression must return a 2D array matching the input image dimensions.

## 5.6 Spectra Math

Applies a per-pixel spectral expression using one or more input images. Each pixel spectrum is wrapped in a Spectrum object (S1, S2, ... up to 20) supporting arithmetic and wavelength access.

Spectrum object attributes:

- S1.spectrum - 1D float64 array of band values for the current pixel

- S1.wavelength - 1D wavelength array

- S1[n] - value at band index n (0-based)

- S1.wavelength2index(w) - nearest band index for wavelength w nm

Standard NumPy ufuncs (sqrt, log, abs, etc.) operate on Spectrum objects. Arithmetic between Spectrum objects is element-wise. The expression must return a Spectrum or 1D array; output band count matches the returned spectrum length. An optional mask layer restricts processing to non-zero pixels.

Expression examples:

| Expression | Description |
| --- | --- |
| S1 + S2 | Element-wise sum of two spectra |
| Spectrum(S1.spectrum / S2.spectrum, S1.wavelength) | Per-pixel spectral ratio |
| Spectrum(np.log(S1.spectrum), S1.wavelength) | Log of spectrum |

Output: Multi-band GeoTIFF. Band count equals the length of the returned spectrum.

# 6. Common Wavelength Ranges

| Application | Start (nm) | End (nm) | Target Features |
| --- | --- | --- | --- |
| Clay minerals (AlOH) | 2100 | 2250 | Kaolinite ~2165, Montmorillonite ~2210 |
| Broad SWIR survey | 2000 | 2500 | General mineral mapping |
| Carbonate minerals | 2300 | 2400 | Calcite ~2340, Dolomite ~2320 |
| Iron oxides | 850 | 1050 | Hematite ~860, Goethite ~920 |
| Vegetation red edge | 680 | 750 | Chlorophyll absorption |
| Vegetation water | 900 | 1100 | Leaf water content ~970 nm |

# 7. Version History

| Ver. | Title | Key Changes |
| --- | --- | --- |
| 0.9 | Spectra Selection Filter | Spectra filter with wildcard, index, and range support. Library browser to list available spectra. |
| 0.8 | Robust ENVI Reader | Direct .hdr parsing and binary reading. Tested against USGS Spectral Library V7 (402 spectra). |
| 0.5 | Unit Auto-Detection | Auto-converts µm ↔ nm wavelength units. |
| 0.4 | Spectral Resampling | Auto-resamples reference spectra to match image wavelengths. Reads wavelengths from image and library. |
| 0.3 | Wavelength Mapping Update | Step 2 uses direct band indices from Step 1 output. |
| 1.2 | Wavelength Bug Fix | Fixed all six new tools not reading wavelengths from ENVI-format rasters. GetMetadata('ENVI') domain tried first, then default domain, then band descriptions. Toolbox group renamed: Step 1 - Wavelength of Minimum v2. Toolbox group renamed: Step 2 - Wavelength Mapping. |
| 1.1 | Six New Spectral Processing Tools | Added Convex Hull Removal (continuum removal, divide and subtract modes). Added Band Ratio (single-band ratio by wavelength in nm). Added Band Ratios (sequential all-band ratios with configurable delta). Added Band Depths (per-band absorption feature depths). Added Band Math (arbitrary NumPy expression over image bands). Added Spectra Math (per-pixel spectral expression using Spectrum objects). |

# 8. References

Kruse, F.A. et al., 1993. The Spectral Image Processing System (SIPS) – Interactive Visualization and Analysis of Imaging Spectrometer Data. Remote Sensing of Environment, 44(2-3), pp.145–163.

Kokaly, R.F. et al., 2017. USGS Spectral Library Version 7. U.S. Geological Survey Data Series 1035, 61 p.

Bakker, W.H., 2010–2022. HyPy – Hyperspectral Python. University of Twente, Faculty ITC.